ENVIRONMENTAL BASELINE STATUS

3.1  	BASELINE DATA

The data on baseline status of six environmental components was collected and compiled from various existing sources and was also supplemented by primary data collected at individual project sites.  A comprehensive database of all data collected is presented in the report  “Environmental Status of MMR, Final Report, May 1997”.   The six environmental components are:

- Air Quality

- Noise Levels

- Ecologically Sensitive Areas

- Socio-economic characteristics of the affected communities

- Health

- Land Use

Both the primary and secondary baseline data were compiled and analyzed during the years 1996 to 1998. The earlier secondary baseline data has now been updated for the years 1998 to 2001 and a comprehensive analysis is presented in this section.  The updated data is mainly for Air Quality and Noise as these are the principal environmental components that would be affected by transport related projects. Among other sources of secondary data, the important ones used are as follows:

- Annual Environmental Status Reports by Municipal Corporation of Greater Mumbai.

- Ambient air quality data monitored by National Environmental Engineering Institute (NEERI).

- Urban Air Quality Management Strategy in Asia, Greater Mumbai Report,  (URBAIR), World Bank Technical Paper No. 381.

- Study for Strengthening the Air Quality Network Operated by MCGB, prepared by Environmental Management Centre Mumbai, 2000.

This chapter describes the prevalent environmental conditions and their trends, mainly for Greater Mumbai, as most of the sub-projects of MUTP are located in this sub-region.  Details of the primary data on air quality, noise and traffic collected as a part of the Sectoral, Programmatic and Micro- Level Environmental Assessments (SLEA, PLEA and MLEA) are described in the relevant sections.

  - ENVIRONMENTAL STATUS OF MCGM

3.2.1	Land Use

Geographically, MMR lies in the North Konkan region, which lies between 18o33’ and 19o31’ north latitude and between 72o45’ and 73028’ east longitude.  The region consists of mainland of North Konkan and two large insular masses of Salsettle – Trombay and Mumbai separated by shallow creeks and tidal marshes.

Map showing the current land use pattern of MMR is given in Figure 3.1.   As seen in the figure, Greater Mumbai consists of the maximum built up area.  Built-up and industrial areas have increased considerably in Vasai – Virar belt along the western corridor and at various towns along the Mumbai – Pune corridor. Environmentally sensitive locations in Greater Mumbai are mainly the coastal stretches, some of which have mangroves and the Sanjay Gandhi National Park of 103 sq. km. having dense forests.

## 3.2.2   Climate And Meteorology

Mumbai has a mean elevation of 11 meters above sea level. The city experiences a tropical savanna climate with monthly humidity ranging between 57-87%. The annual average temperature is 25.3 0C rising to a maximum of 34.50C in June and a minimum of14.30C in January. Average annual precipitation is 2,078 millimeters with 34% falling in the month of July.

The dispersion of air pollution emissions in the region is influenced by wind conditions (direction and speed), mixing heights, stability of atmosphere, collectively termed as the Meteorology of Mumbai’s airshed. Meteorological data is therefore essential to understand spatial-temporal distribution of pollutants.

Meteorological stations of Indian Meteorological Department (IMD) are located at Santacruz and Colaba. The measurements include measurements of wind speed and direction, humidity and temperature and vertical temperature profile recorded by sending a balloon, two times a day.   Table 3.1 provides an analysis of wind pattern in Mumbai.

FIG. 3.1 LANDUSE MAP FOR MMR

Table 3.1 Analysis of Wind patterns in Mumbai

| Summer |  | Monsoon |  | Winter |  |
| --- | --- | --- | --- | --- | --- |
| Pre-Noon | Post-Noon | Pre-Noon | Post-Noon | Pre-Noon | Post-Noon |
| North-South changing to North-east Constant direction of North-West in May. Lowest speed is of 6 kmph and range is between 6 to 8 kmph (fairly narrow range) | North-West to North-South Constant direction of North-West in May. Wind speeds vary typically between 6 to 16 kmph Peaks at 16 kmph and then decreases | South-West and West are the dominant directions during monsoon. Wind speeds are 10 to 12 kmph. September shows a turn in the direction to North-West. | Again South-West and West are the dominant directions. Wind speeds are between 14 to 16 kmph. In June, between 12 to 16 hrs, North-West is also dominant. September shows a turn in direction to North-West. | Direction is between North and North-East that moves gradually to North-East by noon. Speeds low between 6 to 8 kmph increasing from midnight to noon | Starting with North-West and staying on with higher speeds of 10 to 14 kmph |

(Source: Final Report on Tasks 1 and 2: study for strengthening the Air Quality Monitoring Network Operated by BMC prepared by Environmental Management Centre, Mumbai, November 2000.)

Note: The directions mentioned indicate the direction from where the wind is blowing.

3.2.3    Ambient Air Quality

## 3.2.3.1		 National Ambient Air Quality Standards

Table 3.2 given below shows National Ambient Air Quality Standards (NAAQS) as notified by the Central Pollution Control Board. Pollutants concerned are sulphur dioxide, nitrogen oxides, carbon monoxide, suspended particulate matter, respirable particulate matter, for residential, industrial and sensitive area.

Table 3.2 National Ambient Air Quality Standards

| Pollutants | Time Weighted Average | Concentration in ambient air |  |  | Method of measurement |
| --- | --- | --- | --- | --- | --- |
|  |  | Industrial Area | Residential Rural & other areas | Sensitive Area |  |
| Sulphur Dioxide(SO2) | Annual Average* | 80 g/m3 | 60 g/m3 | 15 g/m3 | - Improved West and Geake Method - Ultraviolet Fluorescence |
|  | 24 hours** | 120 g/m3 | 80 g/m3 | 30 g/m3 |  |
| Oxides of Nitrogen as NO2 | Annual* | 80 g/m3 | 60 g/m3 | 15 g/m3 | - Jacob & Hochheiser Modified (Na-Arsenite) Method - Gas Phase Chemiluminescence |
|  | 24 hours** | 120 g/m3 | 80 g/m3 | 30 g/m3 |  |
| Suspended Particulate Matter (SPM) | Annual* | 350 g/m3 | 140 g/m3 | 70 g/m3 | High Volume Sampling, (Average flow rate not less than 1.1 m3/minute) |
|  | 24 hours** | 500 g/m3 | 200 g/m3 | 100 g/m3 |  |
| Respirable Particulate Matter (PM10) , size less than 10 m) | Annual* | 120 g/m3 | 60 g/m3 | 50 g/m3 | Respirable particulate matter sampler |
|  | 24 hours** | 150 g/m3 | 100 g/m3 | 75 g/m3 |  |
| Lead (Pb) | Annual* | 1.0 g/m3 | 0.75 g/m3 | 0.50 g/m3 | ASS Method after sampling using EPM 2000 or equivalent Filter paper |
|  | 24 hours** | 1.5 g/m3 | 1.00 g/m3 | 0.75 g/m3 |  |
| Carbon Monoxide (CO) | Annual* | 5.0 mg/m3 | 2.0 mg/m3 | 1.0 mg/m3 | Non dispersive infra red Spectroscopy |
|  | 24 hours** | 10.0 mg/m3 | 4.0 mg/m3 | 2.0 mg/m3 |  |
| Ammonia | Annual* | 100 g/m3 | 100 g/m3 | 100 g/m3 | -- |
|  | 24 hours** | 400 g/m3 | 400 g/m3 | 400 g/m3 |  |

*	Annual Arithmetic mean of minimum 104 measurements in a year taken twice a week 24 hourly at uniform interval.

** 24 hourly/ 8 hourly values should be met 98% of the time in a year. However, 2% of the time, it may exceed but on two consecutive days.

NOTE:

- National Ambient Air Standards: The levels of air quality with an adequate margin of safety, to protect the public health, vegetation and property.

- Whenever and wherever two consecutive values exceed the limit specified above for the respective category, it would be considered adequate reason to institute regular/continuous monitoring and further investigations.

### 3.2.3.2	 Primary Baseline Ambient Air Quality Monitoring

AIC Watson consultants monitored baseline ambient air quality in 1996 at several locations in the 4 sub-regions of MMR (Island city, Eastern suburbs, Western suburbs and Rest of MMR), as a part of Sectoral Level (SLEA), Programmatic (PLEA) and Micro-level (MLEA) Environmental Assessments. The monitoring details are presented further given in Chapters 4 and 6.

### 3.2.3.3 Air Monitoring By Municipal Corporation Of Greater Mumbai (MCGM)

(A)	Ambient Air Quality in Residential and Industrial Areas

Major air pollution sources in Greater Mumbai are shown in Table 3.3:

Table 3.3 Sectorwise Emission Load of Greater Mumbai

| Sources | Percentage Contribution |  |
| --- | --- | --- |
|  | 1998-99 | 1999-2000 |
| Domestic Sector | 1.52 | 1.5 |
| Industrial and commercial sector | 37.15 | 36.77 |
| Transport | 61.33 | 61.73 |

Source: ESR of MCGM for respective years.

With changing economic structure (declining manufacturing activities) and increasing number of motorized vehicles, transport sector has emerged as the major contributor of air pollution. This trend is likely to continue.

MCGM monitors ambient air quality at various residential and industrial areas for SO2, NOX, SPM and NH3.  MCGM used to conduct ambient air monitoring at 18 monitoring sites spread all over Greater Mumbai till 1999. Figure 3.2(A) shows air-monitoring locations before year 1999. Each location was monitored at least once a week throughout the year for the above four pollutants.  MCGM also publishes Environmental Status Reports for each financial year with statistics on various environmental parameters. In January 2000 the ambient air monitoring stations were reduced to 6 locations with sampling frequency of twice a week to meet regulatory requirements of national Ambient Air Quality Standards.

Air Quality Trends at Specific Locations

Monitoring results for the years 1998-99, 1999-2000 and 2000-01 were compared to determine air quality trends in these years.  As monitoring was done only at 6 locations in the year 2000-2001, only these locations are selected for this analysis. These air-monitoring locations are presented in Figure 3.2(B).

Fig. 3.2(A) Air-Monitoring Locations before Year 1999

Fig. 3.2(B) Current Air-Monitoring Locations in MCGM Area

Summarized annual average data for 6 locations is presented in Table 3.4.  Histograms of these results are presented in Figures 3.3 to 3.5. The following conclusions can be drawn from these results:

- Annual average for SO2  varied between 6 to 36g/m3  in MCGM area. Highest SO2 was recorded at Tilaknagar, located north of Industrial belt of Chembur. It is under the influence of higher level emissions from industries in Chembur and fuel burning in slum area and thus shows highest SO2 level. NOx annual average was recorded between 13 to 69 g/m3 in MCGM area. There was no specific trend observed for NOx but at Khar and Maravali the annual values cross the limits. The higher NOx levels at Khar are attributed to heavy traffic while for Maravali it is attributed to industrial emissions in Chembur. During all the three years SO2 and NOX values at remaining locations are well below the annual NAAQ standards (60 g/m3 for residential areas).

- Suspended particulate matter values varied between 154 to 388 g/m3.  The highest values were recorded at Maravali that is located in the industrial belt near Chembur in the eastern suburbs. At all locations the SPM annual values exceeded the NAAQ annual standards i.e. 140 g/m3 for all the three years.

- Annual average values for ammonia ranged between 41 and 361 g/m3, these levels are well below the annual NAAQ standards (100 g/m3) at all stations except Maravli. The highest level of 361 g/m3 was observed at Maravali Station located in the industrial belt near Chembur in the eastern suburbs, which may be due to proximity to Fertilizer Complex.

# Table 3.4 Ambient Air Quality at Specific Locations for Last Three Years

| # Locations/ ( Type) | ## Parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SO2 (g/m3) |  |  | NOx (g/m3) |  |  | NH3 (g/m3) |  |  | SPM (g/m3) |  |  |
|  | 1998-1999 | 1999-2000 | 2000-2001 | 1998-1999 | 1999-2000 | 2000-2001 | 1998-1999 | 1999-2000 | 2000-2001 | 1998-1999 | 1999-2000 | 2000-2001 |
| Worli (C) | 23 | 30.26 | 22.16 | 35 | 34.3 | 31.25 | 67 | 79.67 | 61.5 | 270 | 218 | 186.1 |
| Khar ( C/R ) | 13 | 20.87 | 16.16 | 48 | 36.35 | 69.25 | 121 | 78.1 | 82.58 | 280 | 306 | 290 |
| Andheri (I/C) | 17 | 26.33 | 16.16 | 29 | 41.48 | 39.33 | 67 | 71.5 | 67 | 219 | 290 | 230.8 |
| Borivali (R) | 6 | 9.09 | 6.91 | 37 | 22.22 | 13.91 | 71 | 54.43 | 41.66 | 252 | 243 | 154.8 |
| Tilaknagar (C) | - | 35.41 | 22.58 | - | 46.21 | 38.75 | - | 82.45 | 82.0 | - | 341 | 282.2 |
| Maravali (I) | 16 | 27.47 | 26.33 | 49 | 46.21 | 64.66 | 330 | 292.6 | 361.1 | 356 | 424 | 388 |

Note: Above yearly averages are taken for financial years (March to April) from ESR of MCGM.

C = commercial; I = Industrial; R = Residential

- An Air Pollution Index (API) based on air pollution standards of the three pollutants has been developed by MCGM combining the three pollutants. The API is computed on the basis of summation of the ratio of the observed concentrations for the three pollutants, SO2, NOx and SPM, to the applicable pollutant specific standard, divided by the number of pollutants.

 ( Observed Value  pollutant specific standard)   X 100

API = 			Number of pollutants

Based on this, API for six areas mentioned above is presented in Table 3.5 below.

Table 3.5   API for six areas in Greater Mumbai

| Location | Type | Air Pollution Index |
| --- | --- | --- |
| Worli | Commercial | 59 |
| Khar | Commercial/Residential | 81 |
| Andheri | Industrial/Commercial | 68 |
| Borivali | Residential | 56 |
| Tilak nagar | Commercial | 67 |
| Maravali | Industrial | 91 |

While pollution in Maravali is due to industrial belt in Chembur, pollution in Khar is mainly attributed to transport sector. Air Pollution Index Descriptor describing the quality of air is given in Table 3.6

Table 3.6 Air Pollution Index Descriptor

| Index Value Range | API Descriptor |
| --- | --- |
| 0 –25 | Good |
| 26 – 50 | Fair |
| 51 – 75 | Moderate |
| 76 – 100 | Unsatisfactory |
| Above 100 | Harmful |

(Source: Pollution Sensitivity Indicator, Environmental Status Report of MCGM, 1999.)

- It can be seen that the ambient air quality has not changed appreciably since the year 1998.  The highest concentrations were observed in 1999 for SPM, SO2 and ammonia.  Whereas, the values recorded in the years 1998 and 2000 did not vary significantly.

#### (B)		Air Quality at Traffic Intersections

MCGM also monitors air quality at traffic intersections for SO2, NOx, PM10 and CO on regular basis. Data at three different intersections is given in Table 3.7.

# Table 3.7 Air Quality at Traffic Intersections

| # Locations | ## Parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SO2 (g/m3) |  |  | NOx (g/m3) |  |  | PM10 (g/m3) |  |  | CO (g/m3) |  |  |
|  | 1998-1999 | 1999-2000 | 2000-2001 | 1998-1999 | 1999-2000 | 2000-2001 | 1998-1999 | 1999-2000 | 2000-2001 | 1998-1999 | 1999-2000 | 2000-2001 |
| Mahim | 16 | 50 | 40.36 | 522 | 419 | 300.0 | 451 | 291 | 244.0 | 10.7 | 9.1 | 5.91 |
| Andheri | - | 98 | 100.9 | - | 520 | 314.4 | - | 520 | 343.0 | - | 11.9 | 7.05 |
| Wadala | - | 62 | 35.45 | - | 446 | 276.1 | - | 338 | 221.0 | - | 5.4 | 3.39 |

Note: Above yearly averages are taken for financial years from ESR of MCGM.

Data Interpretation

- Annual average for PM10 at traffic intersections varied between 221 and 520 g/m3.  Highest value (533 g/m3) was recorded in March 2000, at Andheri west. Lowest PM10 value was recorded in August 2000 at Mahim as 116 g/m3.  These values are quite high and well above the PM10 standards for residential and industrial areas (See Table 3.2 for standards).

- Annual average of Carbon monoxide at traffic intersection is 5.4 to 11.9 g/m3.  Highest (11.9 g/m3) was recorded at Andheri, which is above the standards prescribed.

- Annual average for sulfur dioxide at traffic intersections varied between 16 to 100 g/m3. Annual average for nitrogen oxide was between 276 and 522 g/m3. The highest value for N0x was recorded at Andheri in winter.

- At Andheri west traffic intersection all parameters were recorded as highest through the year 2000-01 and exceeded the standards.

- The annual average values at traffic intersections for all pollutants were generally seen to be lower in the year 2000-01 as against 1999-00.

### 3.2.3.4	Statistical Study by EMC, Mumbai on MCGM Data

The Environmental Management Centre (EMC), Mumbai carried out additional statistical analysis, while conducting the study for strengthening Air Quality Monitoring network of MCGM. This analysis, based on the air quality data for 1997 -1999 shows that on an average ambient air quality has not changed appreciably since the year 1998.  However, the variations within the region are quite high as could be seen from Fig 3.6, 3.7 and 3.8. These figures present deviation from regional means for SO2, NOx and SPM.

The annual mean values for Greater Mumbai (calculated as average of annual means of all monitoring stations in Greater Mumbai) for SO2, NOx and SPM for the year 1997, 1998 and 1999 are given in Table 3.8.

Table 3.8 Yearwise Regional Mean Values of Pollutants for Greater Mumbai

| Year | SO2 | NO2 | SPM |
| --- | --- | --- | --- |
| 1997 | 36 | 24 | 270 |
| 1998 | 40 | 25 | 260 |
| 1999 | 38 | 25 | 270 |

Unit: g/m3

Air Quality Trend Analysis

The air quality trend analysis as reported in “The study for strengthening the air quality monitoring network operated by MCGM, is summarized in the following paragraphs.

- There is a statistically significant downward trend for SPM over the period 1997 to 1999 at Mulund, Bhandup, Ghatkopar, Mankhurd, Aniknagar, Sion and Worli.  However, almost at all the stations in Mumbai there is a considerable exceedence over standards for SPM, the average percentage of exceedence being 70%.

- In the case of NOx, no station reports a statistically downward trend. Two stations viz. Supari Tank and Mankhurd show statistically upward trend during 1997-99.  Again stations such as Khar, Sion and Maravali show exceedences; which may be attributed to increase in vehicular traffic.

- The monitoring stations at Maravali, Supari Tank, Andheri and Jogeshwari show a statistically upward trend for SO2, but the exceedences over standards are only marginal, being of the order of 5 to 10%.

- From the time series representation of the trend of regional means (based on annual averages for the three pollutants NOx, SO2 and SPM it is seen that the past trend is more or less continuing, except that the mean values have decreased marginally in the past 3 years.

Seasonal Levels of SO2

The levels of SO2 recorded at different sites in Mumbai are high during winter months as compared to those in monsoon. The levels are higher in winter by 1.2-8.7 times as compared to those in monsoon season, maximum increase recorded at Babula Tank which is 8.7 times higher and minimum increase found to be 1.2 times higher at Jogeshwari.

Seasonal Variation of NOx

The levels are high during winter as compared to those in monsoon. The levels are higher by 2.3 to 7 times during winter and maximum increase was recorded at Bhandup and minimum increase was recorded at Sakinaka.

Seasonal Pattern

SPM levels are high during winter as compared to those in monsoon. The levels are higher in winter by 1.7-2.6 times. The maximum increase was recorded at Supari Tank and minimum at Babula Tank site.

Please refer to Fig 3.9 for the values of SO2, NO2 and SPM over the years.

3.2.3.5	Ambient Air Monitoring by NEERI

National Environmental Engineering and Research Institute (NEERI) a Govt. of India organization has been conducting ambient air quality monitoring at some locations in Mumbai as a part of the National Ambient Air Quality Monitoring Program (NAAQM).  The monitoring has been carried out for different parameter such as SO2, NOx, SPM, PM10, H2S, NH3 etc. The annual average data is summarized in the form of graphs in Fig. 3.10 to 3.12 for Kabladevi, Parel and Bandra locations respectively. Specific comments are as follows:

- SO2 and N0x are in general well below the air quality standards at all locations

- SPM levels exceed the annual average residential area standards of 140 g/m3. Highest values were observed in 1997 (except Parel).  Since 1997 the SPM levels show a decreasing trend.

- PM10 levels too exceed the annual average residential area standards of 60 g/m3.  Here too the highest values were observed in 1997 (except Parel).  Since 1997 the PM10 levels show a decreasing trend, although they continue to exceed the standards.

3.2.3.6  Salient Features of URBAIR Study

The URBAIR study for Greater Mumbai was taken up as a part the URBAIR program (A World Bank funded project) which aimed at improving air quality of Metro Manila, DKI Jakarta, Greater Mumbai and Kathmandu Valley. The URBAIR study lays emphasis on developing Air Quality Management strategy, which takes into account the costs and benefits of proposed actions identified for abating air pollution. The study used the following steps to develop an Air Quality Management strategy for Greater Mumbai.

- Air quality assessment

- Environmental Damage Assessment

- Abatement options assessment

- Optimum control strategy

Assessment were based on monitoring data from MCGM - network of 22 stations, GEMS network operated by NEERI and Maharashtra Pollution Control Board (MPCB)  and various other monitoring sites inside and outside Mumbai.

Major Baseline Findings Were as follows:

- The average total suspended particle (TSP) concentration in Mumbai region has increased considerably since 1980 from 200 g/m3 to 250 g/m3 in 1991. Maravali and Chembur Naka showed highest TSP levels. Total annual emission of TSP was estimated to 32,000 tons / year.

- PM10 has not recently been measured in Mumbai. The study by WHO in 1985 on ‘human exposure to respirable particles’ indicated that the concentrations of, and exposure to PM10  in Mumbai in 1982 were much higher than the WHO air quality guideline. Total annual emission of PM10 was estimated to 16,000-tons/ year.

- As per MCGM monitoring (1980 – 87) annual average levels for lead ranged between 0.5 to 1.3 g/m3 , which exceeded the WHO guidelines. From 1980 to 1987, average concentration in the air nearly doubled.

- NOx increased by about 25 % while, sulfur dioxide concentration declined due to increased use of natural gas and low sulfur coal.

3.2.3.7	Overall Conclusions

All above studies point out that SO2, NOx are generally well below the National Ambient Air Quality Standards (NAAQS) at residential and industrial locations. N0x levels however substantially exceed the standards at traffic intersections, as shown by monitoring done by MCGB.  Also SO2 and N0x levels have not changed significantly since 1996 when ambient air quality was monitored for SLEA, MLEA and PLEA purposes.

All studies have shown that PM10 and SPM exceed the NAAQS at almost all residential and industrial locations and also at traffic intersections. For both SPM and PM10, the highest values were observed in 1997 (except Parel).  Since 1997 both pollutants show a decreasing trend, although they continue to exceed the standards.  It is clear, that a comprehensive strategy is needed to bring levels of SPM and PM10 within levels of NAAQ standards gradually over the years.

Values of Regional Mean

Values of Regional Mean

Regional  Mean

3.2.4   Noise Quality

Increasing population and traffic densities have led to increased levels of noise. Noise is the result of a number of activities such as road traffic, aircraft, railways and industrial and commercial activities. Noise due to traffic (including hawkers) is the predominant source of irritation.

Noise on account of public processions, festivals, marriage parties, political meetings and such other activities is also substantial.

Public places like railway platforms, bus terminals and even airport terminals are also excessively noisy. Auto rickshaws, two  wheelers and many diesel vehicles generate high noise levels.

The range of noise levels observed in Mumbai in the years 1997 –99 are represented in Table 3.9.

# Table 3.9 Ambient Noise Quality

| # Area | Noise levels(dB) In Mumbai |  |  | C.P.C.B. Standards |  |
| --- | --- | --- | --- | --- | --- |
|  | 1997 | 1998 | 1999 | Day dB/A | Night dB/A |
| Residential | 55 - 85 | 57 - 82 | 46 - 113 | 55 | 45 |
| Commercial | 62 - 86 | 72 - 80 | 58 - 113 | 65 | 55 |
| Traffic near Industrial Area | 70 - 88 | 72 - 88 | 70 - 103 | 75 | 70 |
| Airport area | 92 - 94 | 78 - 90 | 86 - 103 | 65 | 55 |
| Silence zone | -- | 54 - 60 | 48 - 85 | 50 | 40 |

Source: ESR of MCGM, 1997-98 to 1999-2000

Noise levels in Mumbai were observed to exceed the CPCB ambient noise standards.

## 3.2.5	Ecological Status

### Coastal Areas

The environmentally sensitive locations in Greater Mumbai are mainly the coastal stretches, some of which have mangroves and the Sanjay Gandhi National Park having dense forests.

### Wetlands and Mangroves

Mumbai’s coastline exhibits beautiful natural mangrove vegetation. The flora consists of Bruguiera, Excoecaria candiria, Sonocia sps. and Sonneratia apetala, Rhizophora mucronata and Avicinia apatala. Avicinia apatala is commonly available around Thane, Diwa, Mahim Creek and Thane Creek.  CRZ Regulations, 1991 provide for protection of mangroves.  As required by the CRZ Regulation, 1991, Coastal Zone Management Plan  (CZMP) for Greater Mumbai has been prepared by GoM and approved by MoEF.  The Coastal Regulation Zones (I, II and III) are shown in Figure 3.13.

### Forests and Wildlife

As per the sanctioned Regional Plan for Mumbai Metropolitan Region Area (1996 –2011) the total forest area is 1090.62 sq.km, which is 25.75% of the total land area. The area can be classified as given below:

These forest resources are under pressure due to illicit felling of trees, encroachment for agriculture and fuel purposes. The National Parks and Sanctuaries located in the MMR are listed in Table 3.10.

The coastal wet lands, forests and natural reserves in MMR are shown in Figure 3.14.

Figure 3.13  Coastal Regulation Zone Map

Figure 3.14 - Coastal Wetlands, Forests And Natural Reserves In MMR

Table 3.10 Details of the Natural Reserves

| Name of the National Park/Wildlife/Sanctuary/ Game Reserve | Area in sq.km | District | Principal Wildlife |
| --- | --- | --- | --- |
| Sanjay Gandhi National Park, Borivali | 103.09 | Greater Mumbai and Thane | Nearly 43 mammals belonging to 8 natural orders and 17 families are reported Animal species in this park include the following eight endangered species, viz, Panther, Rusty spotted cat, Jungle cat, Jackal, Small Indian civet, common Palm civet, four horned Antelope and Mouse deer. Park has vegetation types ranging from mangrove forests to semi-evergreen forests which contains various economical, medicinal aromatic and ornamental plant species The Kanheri caves, located within the park are of high archaeological value. |
| Karnala Bird Sanctuary | 4.48 | Raigad | The Sanctuary has southern tropical deciduous vegetation Known for 150 different species of birds like spotted dove, black winged kite, tailor birds, white backed Vulture, etc. |
| Mahim Nature Park | 0.15 | Greater Mumbai | About 40 varieties of migratory birds visit every season. |

## Socio-Economic Characteristics

The report on Regional Plan for Mumbai Metropolitan Region, 1996- 2011, based on a multi purpose household survey carried out in 1990, presents the following socio economic characteristics of the population of MMR:

Demographic and Socio-Economic Profile of MMR

The average household size in Urban MMR is 5.1 with a sex ratio of about 880 females per 1,000 males.  About 60% of households have between 4 and 5 members while about 20% are single member households.  Navi Mumbai has the lowest average household size of 4.3.

Mother tongue of nearly 50% of the households is Marathi followed by Gujarati (15%), Hindi (9%) and Urdu (6%).  Western suburbs of Mumbai display a highly cosmopolitan nature.

About 46% of the households are migrants.  Of these, about 20% have settled in Mumbai during the past 10 years.  Migrants with their origins in rural areas constitute about 65%.  About 57% of the migrants are from outside Maharashtra.

Unemployment is not a serious problem.  Only about 5% of the total work force are on the lookout for employment.  The average number of earners per household is 1.7.  Among all households the average income is Rs.2, 600 per month.  Assuming a monthly income of Rs.1, 000 as the poverty level, about 23% of the households are in this category against 7% who have an income of over Rs.5, 000 per month.

Health

The environmental health status of Greater Mumbai is largely affected by the fact that half of its population lives in slum colonies or pavements. Slum dwellers suffer from the exposure to air pollution in form of smoke, foul smells from municipal garbage & dumping grounds, traffic pollution and noise pollution. It is observed that majority of the slum and pavement dwellers are susceptible to diarrhoea, dysentery, typhoid, jaundice, colds and coughs, flu, bronchitis and asthma.

The health status of the general public has also been reported to be affected in industrial and congested traffic areas of Mumbai. Notable among these is Chembur area with heavy industries. Malaria is perceived as a major health-related problem.  As regards other ailments, Gastro-intestinal disorder, pregnancy related and respiratory problems are acknowledged as the more common ones.  The causes for the health problems are attributed to open drains mosquito menace and to air pollution.  Accumulated garbage is also perceived as an important factor leading to poor health.

Studies in Mumbai have shown that effect of gases and particulate matter from vehicles and industries is manifested in the respiratory diseases in the people either working in or staying near the source of pollution.

Socio- Economic Characteristics of PAHs

MMRDA has prepared a Resettlement Action Plan (RAP) for the PAHs to alleviate the adverse social impacts associated with the implementation of MUTP. In MUTP about 19000 families are likely to be affected and will require settlement.

Out of the total number of 19228 PAHs, baseline survey in case of eight subprojects involving 15267 PAHs has been completed.  For the remaining sub projects the survey work is in progress.  From the available data the principal characteristics of the PAHs can be deduced as follows:

- More than 99% of the PAHs are squatters and do not have any tenurial rights of the land they occupy.  Less than 1% of PAHs are legal owners or tenants of the land or buildings they occupy.

- The monthly mean household income is Rs.2943, which is 1.8 times the official poverty line for the urban household, 35% of the households are below the poverty line.

- The average household size is 4.06. Nearly 13% of the households are women-headed.  The percentage of workers to total population is 36%.  Out of the total workers, a third are self-employed and a sixth are engaged as casual labour.

- Most of the squatter settlements have drinking water through stand posts, sanitation facilities are by way of community toilets but inadequate in numbers.  The maintenance of these facilities is poor.  Lack of storm water drainage subjects some of these settlements to flooding during monsoon.  Collection of solid waste is not satisfactory.  Settlements near railway suffer noise pollution and pose a threat to the lives of the residents and to the safe passage of trains.  Generally the standards of health and hygiene leave much to be desired.

- Less than 2% of the PAH belong to the Sc8hedule Tribes.  However, that itself is not an indication of their being “indigenous people”.  Though they might have originally belonged to tribal communities in the distant past, as of now they are integrated with the city life and do not have their traditional habitat or follow traditional ways of life.  The social impact of the project on such people would therefore be similar to other PAH’s.  In any case there is general reluctance to reveal the caste or tribe during surveys and data on tribal origins of PAH’s has not been included the baseline socio-economic surveys.

Additional details of demographic, social, economic and environmental characteristics based on the baseline survey of 11760 PAHs (out of which 10493 are resident households) carried out so far are given in Appendix 3.1.

Environmental Status of MMR  by AIC Watson Consultants, Mumbai,  May 1997

Environmental Status Report of MCGM for the years 1997-98, 98 –99 and 99-2000.

URBAIR - Urban Air Quality Management Strategy in Asia, World Bank Technical Paper No. 381, 1997.

Final Report on Tasks 1 and 2: study for strengthening the Air Quality Monitoring Network Operated by

BMC prepared by Environmental Management Centre, Mumbai, November 2000.

Environmental Status Report of MCGM for the years 1997-98, 98 –99  and 99-2000.

URBAIR - Urban Air Quality Management Strategy in Asia, World Bank Technical Paper No. 381, 1997.

CRZ I – a) Areas that are ecologically sensitive and important; b)Area between the Low Tide Line and

High Tide Line.

CRZ II – The areas where buildings and infrastructure have already been developed.

CRZ III Areas that are relatively undisturbed and those which do not belong to either category I and II.