APPENDIX 4.1

SECTORAL LEVEL ENVIRONMENTAL ASSESSMENT (SLEA) METHODOLOGY

1.1	GENERAL

A Sectoral Level EA is an evolving methodology and has been found to be very useful to examine the cumulative impacts of multiple projects planned in the same sector. It usually addresses the mixture of projects proposed for the next few years which may comprise a number of large and small projects, the later not warranting individual environmental assessments. They should result in identification of major environmental issues in the sector and development of database enabling projects specific EA to proceed expeditiously.

A variant of this application called a programmatic level EA that addresses a number of small projects which can be replicated at various locations and for which the impacts are more or less the same at any location.

Other purposes of SLEAs are:

- Review of environmental impacts of sector investment alternatives

- Evaluation of the environmental effects of sector policy changes

- Assessment of:

-	Requirements in the sector for environmental review mitigative measures and EMP

-	Capacities of Sectoral institutions

-	Need for training, development of guidelines for institutional

Strengthening.

A Sectoral level strategy is formulated to guide the development of an economic or service sector for the benefit of the public, and aims at achieving the objectives set forth for this purpose. Traditionally, a strategy is developed on the basis of the techno-economic evaluation of various alternatives or options.    Each option is defined by a set of actions or projects which are expected to translate the underlying theme of the option into reality.    An option or a mix of options, which is determined to be cost-effective in achieving the specified objectives is then selected as the strategy to be pursued for the development.

In evaluating these alternatives or options, little consideration is given to their environmental or social (other than explicit sectoral objectives) implications. With increased significance of interaction among social, environmental and economic sectors, the evaluation of alternatives/options must also consider their impacts on environmental components, society at large and the stake holders. These options or alternatives must be further examined with respect to the institutional feasibility of implementing various projects identified under each of these options. Also, measures to alleviate adverse impacts associated with options or a strategy should be identified for guiding the implementation of the selected transportation strategy.

The sectoral level assessment of MUTP under this assignment is perhaps the first of its kind being carried out in India and so is the environmental assessment methodology evolved for the purpose to analyze the four transportation options described in the previous section. This methodology comprises the following elements.

- Scoping or identifying spatial and temporal boundaries for analysis.

- Identifying various environmental component (areas) and stakeholders likely to be impacted under a transportation option.

- Identifying indicators or indices which can quantify both adverse and

beneficial impacts to the extent possible.

- Identifying impacts, which cannot be quantified, and which should be

described qualitatively.

- Developing techniques and / or identifying tools and models to make future projections / predictions so that the options could be evaluated with respect    to different time horizons.

- Identifying or developing an appropriate method for comparing different types of impacts across the options so that the selection of an appropriate transportation strategy can be facilitated.

- Examining the implementation of the options or alternatives from the

Institutional point of view.

- Identifying the mitigation measures for the adverse impacts likely to result from the implementation of the options.

It should be noted that the above elements are not sequential steps for carrying out an environmental analysis. Nevertheless, these elements form the basis for conducting SLEA.

This section presents the methodology for carrying out SLEA. Accordingly, the rest of the section is organized to present scoping i.e. spatial and temporal boundaries for carrying out the analysis: environmental impact areas or components; and methodology used for estimating the impacts on each environmental, component. Indicators or indices used for quantifying the impacts are also presented, wherever possible. In such cases, where impacts can not be easily quantified, qualitative description of impacts in a structured manner is emphasized.

1.2	SCOPING

For the purpose of the environmental analysis, MMR is divided into four sub regions namely Island City, Western Suburb, Eastern Suburb and the rest of MMR. These four sub regions have been considered because of their distinct land use, economic characteristics and transportation characteristics.    For example, residential and industrial land use in Eastern Suburbs generates heavy vehicle movement or truck traffic in the sub region, whereas traffic in Western Suburbs is dominated by private vehicles.      The land use characteristics of the sub regions are shown in Table 4.1.1.    Figure 3.1 in chapter 3 shows the land use characteristics of the entire MMR.

Table 4.1.1 Major Land Uses in the Four Geographic Zones of MMR

| Geographic Zones of MMR | Characteristics of the Major Land uses |
| --- | --- |
| Island city | Residential and Commercial |
| Eastern Suburbs | Residential, Industrial and wetlands |
| Western Suburbs | Residential, Industrial, Forests and ecologically sensitive wetlands |
| Rest of MMR | Residential, Industrial, Forests, Wetlands and Agricultural lands. |

Whereas, an environmental analysis is carried out for the above four sub regions to understand the implications of various options on each sub region individually, the methodology for carrying out analysis cuts across all these sub regions. Further, the analysis is done for two planning time horizon years 2001 and 2011.

1.3	ENVIRONMENTAL IMPACT AREAS

A transport alternative or option comprises various projects and programs, implementation of which is expected to achieve the transportation goals of MMR in consonance with the underlying theme of the option, e.g., emphasis on promoting public transportation or discouraging traffic flow in a specific sub region. Each alternative could influence the composition and volume of traffic to varying degree depending on the type of projects and programs envisaged under the alternative.    However, basic modes of traffic such as cars, taxis, two wheelers, auto rickshaws, buses and trains will continue to exist in MMR under each of the alternatives.    Since, the operation of these vehicles results in air pollutant emissions and noise, impacts of each transport alternative on the air quality and noise levels are considered in the ' environmental analysis.

The implementation of the transportation projects will involve the construction of roads and railway lines through or adjacent to ecologically sensitive areas. Hence, ecological impacts of the alternatives are also considered in the analysis. The project implementation may further result in direct or indirect social impact. The direct social impacts could arise from the displacement of people and commercial establishment from the road and rail alignment areas. The ease of commuting due to a new or modified road / rail link would, however, be beneficial to the commuters and goods traffic. The secondary development that may come up due to a rail/road link could also have social implications.

Hence, the four impact areas considered for the environmental analysis of the transportation alternatives are as follows.

- Air Quality Impacts - Noise Impacts - Ecological and Land Use Impacts - Social Impacts (Limited to Displacement of People)

Although each of the transport options will affect the above mentioned four environmental components to varying degree, the level of service it renders will also vary from option to option.    In order to select the option not only from environmental angle, but also from the service point of view in additional impact areas namely transport service has also been considered.

Since, air quality and noise impacts of various transport alternatives are determined by traffic composition, volume and speed, these traffic parameters for different alternatives and sub regions are estimated by using the Regional Transport Model for MMR. In order to appreciate how different development scenarios and transportation alternatives influence the determination of the above traffic parameters, a brief overview of the Regional Transport Model (developed by WS Atkins for MMR) is given below. This is followed by the methodology used for determining the above impacts.

1.4	THE STRATEGIC TRANSPORT MODEL

The Strategic Transport Model for MMR, developed as a part of the WS Atkins study, predicts traffic volumes and speeds for different vehicle modes for each transportation link under various transportation alternatives, for the base year (1996) and year 2011.

The model considers three modes of travel i.e. cars and motorcycles: taxis and auto rickshaws; and public transport (bus and rail). The MMR is divided into 110 zones (47 in Island City, 27 in the suburbs and 36 in the region outside MCBM for developing transportation networks to be used in the transport model. There are 1200 major road links considered in the model. The model generates separate networks for private vehicles (cars and motor cycles), taxis and autos rickshaws and public transport for year 1991.    The latter includes 500 bus and    rail routes.    The two important sub models, which allow the transport model to predict future transport scenarios and associated • traffic parameters are:

- trip-end submodel

- joint trip distribution / modal split submodel

Trip-end Submodel:

This sub model relates demand for travel - trip generation and trip attraction i.e. for each zone to zonal planning parameters.      Peak and off-peak periods are simulated separately.      The set of equations used to simulate trip generation and trip attraction for different transport modes is shown in Table 4.1.2.

The dependence of trip generation and trip attraction on planning parameters such as House Holds owning Vehicles (HHV), Resident Workers (RW) and Employment places (EMP) makes the transportation model responsive to various planning strategies. For using this submodel for predicting generation trips and attraction trips for various zones in 2001 and 2011, appropriate projection tools for estimating HHV, RW and EMP need to be used.

Joint Trip Distribution/modal Split Submodel :

This submodel predicts the pattern of different transport modes in MMR. The assumption underlying the sub model is that an individual selects a transport mode based on the perceived cost of travel by different traffic modes, which also internalizes value of time. Therefore, other factors remaining constant, "the number of trips between any two zones (i,j) by a particular mode m

- increases as trip generation in i and attraction in j increases

- decreases as the cost of travel by mode m between i and j increases, and

- decreases as the travel cost by mode m increases relative to other modes.

The submodel is represented by the following equation.

Tij = r, Gi.Si.Aj.Fil Where, Tij = number of trips between zone i and zone j Gi = total generation trip-ends in zone j Aj = total attraction trip-ends in zone j Si, ri = balancing factors.

Table 4.1.2 Trip-End Submodel Equations

i) Private vehicle trips

Peak period

Generation = 433 + 0.949 HHV (r2 = 0.587, std Error = 1044, t (HHV) = 9.5) Attractions = 25.6 + 0.001 EMP1.32 (r2 = 0.595. std Error = 4487. t = 4.4)

Off-peak period

Generation = 1349.38 + 1.0938 HHV (r2 = 0.372, std Error = 1790. t (HHV) = 6.33) Attraction = 2.35 + 0.0000436 EMP2.15 (r2 = 0.631, std Error= 7388, t = 5)

ii) Taxi trips

Peak period

Generation / Attraction = 220 + 0.004 EMP2.5 + 0.378 HHV

(r2 = 0.707, std Error = 2136. t (HHV) = 2.7, t(EMP) = 44.2)

Off-peak period

Generation / Attraction = 28.749 +0.154 EMP2.15+1.7129 HHV

(r2= 0.72, std Error = 9792. t (HHV) = 2.65, t(EMP) = 4.36)

iii) Public transport trips

Peak period

Generation = 15.7 + 0.566 HHV + .335 RW

(r2 = 0.959, Std Error = 2225, t (HHy) = 2.6, t(RW) = 36.2)

Attractions = 111.9 + 0.0097 EMP 1.32

(r2= 0.871, std Error = 20070, t= 4.4)

Off-peak period

Generation = 2260 + 1.329 HHV + 0.242 RW

(r2 = 0.87, Std Error= 3060, t (HHV) = 4.5. t(RW) = 17.94)

Attraction = 0+0 65078 EMP (r2 =0.73, Std Error = 22494, t = 12.13)

(Source: Comprehensive Transportation Study for MMR, Tech. working Paper No. 6)

Where,

HHV - HouseHolds owning Vehicles: RW - Resident Workers:

EMP - Employment places, and

ri. Si = balancing factors (constants)

Fij = ke- Bcij Cij a = the deterrence function

where,

k          = cost factor (a constant)

Cij, = generalized cost factor

B = calibration constant - exponential function

a = calibration constant - power function

These trips are assigned to each road link based on traffic speed curves relating traffic flow (PCU) to speed for private or non-public transport vehicles. Bus speeds are modified to account for delays caused during bus stoppages. Traffic speed curves are developed for the following road types with different carrying capacities.

- Urban - single carriage.way

- Urban - dual 2/3 lane carriage way

- Urban - dual 4-lane carriage way

- Rural all purpose.

Further, centroid of each zone is connected to appropriate nodes of the road network for non-public modes and to bus stops and stations for public transport modes.

The Strategic Transport Model provides output in the form of PCUs, vehicle speed and vehicle kms for heavy and light vehicles for each road link. This output has been processed to reassign the vehicle kms to the four identified sub regions for carrying out the environmental analysis. Also, since pollution generation potential of each vehicle type i.e. heavy vehicles, cars, auto rickshaws and 2 wheelers is different, non-public transport PCUs and Public transport PCUs have been converted back into various transport modes for the base year and future planning horizon years.    In the present case, year 2011 is considered to be the future-planning horizon and accordingly the assessment is done for this year. The assumption underlying this conversion is that the traffic composition in future years will remain the same as for the base year (1996).

The traffic parameters predicted by the Regional Transport Model for different transport alternatives are used as inputs for air quality and noise impact analyses.

1.5	AIR QUALITY IMPACTS

Air quality impacts of various transportation alternatives on a sub region result from emissions generated by different types of vehicles constituting traffic in that sub region.    The type of a vehicle determines the nature and level of pollutants present in its exhaust.      Petrol driven light vehicles (cars, two wheelers, and auto rickshaws) mainly discharge carbon monoxide and hydrocarbons. Two wheelers and auto rickshaws give rise to relatively high levels of pollution due to the incomplete combustion of fuel in their two stroke engines. Diesel driven heavy vehicles predominantly emits particulate matter and nitrogen oxides.    Depending on the sulphur content of the fuel minor levels of SO; may also be present in the exhaust.

Since the implementation of each transportation alternative would generate different traffic volumes and composition in the four sub regions for year 2011, the resulting air quality impacts associated with the alternative would be different.    Hence, in order to determine the air quality impacts associated with a given transportation alternative for the horizon year, dispersion of vehicular emissions need to be simulated to estimate the resulting ambient pollutant concentrations of CO, NOx, PM, Pb and S02 for each sub region. The predicted pollutant levels for the year 2011 under each transportation alternative when compared to the base year pollution levels provide an assessment of air quality impacts due to the alternative.    Also, for the quantification of impacts, air quality indicators or indices need to be identified or developed. Hence, air quality impact analysis involves the following steps.

- Determining baseline air quality (reflecting air pollution resulting from the base year traffic level) of the sub regions,

- Estimating vehicular emissions in each sub region for each option / alternative for 2011,

- Simulating dispersion of the vehicular pollutants to estimate resulting ambient concentration increments for each sub region and option for the year 2011,

- Assessing impacts of the estimated concentration increments on various stakeholders or target groups by using value function curves for the above pollutants. (These curves provide a relationship between ambient pollutant concentrations and the resulting impacts on human health and the environment).

1.5.1	Baseline Air Quality

The baseline or base year air quality of the four sub regions and MMR as a whole is used to assess change in the air quality due to the transportation alternatives for the horizon year 2011.      For making assessment of the baseline air quality of each sub region,    the sub regions were divided into square grids. Air pollution monitoring was carried out at various locations in each of the four sub regions.    The pollutants monitored at most of the locations or sites included sulfur dioxide, nitrogen oxides, particulate matter, lead, PM10, carbon monoxide and hydrocarbons. The sites were selected to represent predominant vehicular pollution areas. The monitoring was carried out for three or more days.    The monitored pollutant concentration values were used to estimate baseline concentrations at the centre of each grid by using the inverse square weighted interpolation technique.    The weighted interpolation scheme incorporates the influence of prevailing wind pattern and topographical features. The estimated pollutant concentration values in a grid were then used to determine quality index for each pollutant using value function curves to represent its potential air quality impacts.      The value function curves for CO, NO, , 802, PM10 and Pb are provided in Figure 1.5.1 to Figure 1.5.5.      Quality index values vary from 0 to 1, higher values indicating better quality.    (Reference : Environmental Impact Analysis - A New Dimension in Decision Making by R.K. Jain and L.V. Urban, 1981).

FIG. 4.1.1 : VALUE FUNCTION CURVE FOR CARBON MONOXIDE

FIG.4.1.2    VALUE FUNCTION CURVE FOR SULFUR DIOXIDE

FIG. 4.1.3    VALUE FUNCTION CURVE FOR PM10

FIG 4.1.4 VALUE FUNCTION CURVE FOR NOx

FIG. 4.1.5 : VALUE FUNCTION CURVE FOR LEAD (Pb)

1.5.2 Air Dispersion Modeling Analysis

An air quality model relates air pollutant emissions to the resulting ambient air pollutant concentrations under different environmental settings including meteorological conditions. The output of the modeling analysis is in the form of pollutant concentrations at the specified receptor locations. The model estimated pollutant concentrations can be used in determining quality indices from value function curves to reflect the air quality impacts of the transportation alternatives.

The choice of air quality model depends on various factors, which include the following:

- Spatial domain on which the model is to be applied

- Time domain on which the model is to be applied

- Nature of pollutants

- Type of pollutant sources

It is difficult to develop or select a model, which takes    into account all these factors in a satisfactory manner.    An attempt is however made to select a model, which meets most of the modeling requirements, and at the same time, is simple to use and calibrate/validate.    The choice of the Roll Back model for estimating pollutant concentrations, instead of the earlier proposed PAL model is made because of the simplicity of the former.    The use of a simple model as against the sophisticated ones is particularly preferred for a sectoral level .analysis as the input requirements for the latter are-too detailed    and not justified for a macrolevel assessment.      Moreover, PAL has the following limitations.

- Although PAL can handle six types of sources including special line and curved sources, it can handle only 99 road links in a single run. There are about 600 road links in the entire region, which need to be considered for modeling purpose.    Further, some of these links traverse through more than one sub region (impact analysis is carried out for each sub region separately as indicated in the scope), which complicates the application of the model to a sub region.

- PAL can be used to estimate pollutant concentrations up to several hundred meters only.    The area under each sub region extends up to several kms.

- PAL inherits all the limitations of the Gaussian equation based models, such as flat terrain assumption, unreasonably high concentration estimates during light wind conditions, etc.

- PAL's input data requirements are intensive and collection of data at such a detailed level can not be justified for a sectoral level analysis.

The above limitations of PAL also suggest the use of a simple model like the Roll Back model for estimating vehicular pollutant concentrations under different transportation alternatives for the year 2011.

Roll Back Model:

Roll Back Model is a simple proportionate reduction model, which is based on the assumption that reduction in pollutant emissions proportionately reduces the resulting ambient air pollutant concentrations.    Thus the model can be mathematically represented by the following equation.

C1/C2 = Q1/Q2

where,

C1 = concentration at a given point due to emissions Q1

C2 = concentration to be estimated at the same point due to emissions Q2

C1 and C2 may also be called as receptor data, whereas Q1, and Q2 are called as source data. The predicted concentration 02 correspond to the same set of meteorological conditions as that for Ci and are of same nature as that of Ci. In the present case Ci represents the ambient air pollutant concentrations and therefore so does C;.' The model predicts the future ambient air pollutant concentrations in the light    of changes in the traffic related emissions (Q;) assuming the other emissions to remain the same for the future years.

For the application of the Roll Back model in the present study, the model equations for the concerned pollutants were developed by using the estimated ambient air pollutant concentrations and estimated vehicular emissions for the base year. The estimation of the base year pollutant concentrations has been discussed in the previous sub section. For estimating base year emissions, the road links traversing through each grid of the sub region are identified. Vehicular emissions are assigned to the grid based on the length of links intercepted by the grid and vehicle kms carried by the intercepted road segments.    Vehicle kms for each traffic mode are estimated by using the Regional Transport Model. Vehicular emissions for the base year in each grid are estimated by segregating the vehicles into old (production before 1991) and new (production after 1991) vehicles and calculating total emissions using emission factors prescribed for 1991 for old vehicles and those prescribed for 1996 for new vehicles. The IIP emission factors, which are prescribed by MOEF for different years, are given in Table 4.1.3. Vehicle statistics in terms of old and new split is obtained from R.T.O.

Table 4.1.3. IIP Emission Factors (gm/km) for various Traffic Modes

| Vehicle Type | 1991 |  |  |  | 1996 |  |  |  | 2000 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOx | Pb | SO2 | CO | NOx | Pb | SO2 | CO | NOx | Pb | SO2 |
| Two Wheeler | 12.0 | 1.58 | 0.009 | 0.65 | 4.5 | 1.58 | 0.009 | 0.65 | 2.0 | 0.658 | 0 | 0.65 |
| Three Wheeler | 18.0 | 2.37 | 0.013 | 0.65 | 6.75 | 2.37 | 0.013 | 0.65 | 4.0 | 0.658 | 0 | 0.65 |
| Car | 14.3 | 1.57 | 0.027 | 1.4 | 8.68 | 1.32 | 0.027 | 1.4 | 2.72 | 0.427 | 0 | 1.398 |
| Bus | 11.25 | 4.0 | 0.0 | 4.3 | 9.0 | 1.2 | 0.0 | 4.3 | 1.6 | 1.78 | 0 | 4.295 |
| Truck | 11.25 | 4.0 | 0.0 | 4.3 | 9.0 | 1.2 | 0.0 | 4.3 | 1.6 | 1.78 | 0 | 4.295 |
| LCV | 9.6 | 2.5 | 0.0 | 1.16 | 7.7 | 2.32 | 0.0 | 1.16 | 1.09 | 1.29 | 0 | 1.165 |

Note :    IIP - Indian Institute of Petroleum

Vehicular emissions needed for estimating pollutant concentrations for the transportation options during 2011 are computed by using the proposed emission standards for the year 2000 as emission factors. (Ref : Report on Vehicle Emissions and Control Perspectives in India prepared by Indian Institute of Petroleum for Ministry of Environment and Forests, Government of India in 1994)

In case of computation of lead emissions it is assumed that petrol cars registered in Brihan Mumbai after 1st April 1995 would have zero lead emission as these cars would be running on unleaded petrol. However, cars registered in the Rest of MMR would continue to have lead emissions in accordance with the 1996 standards. All the cars manufactured after 2000 are assumed to have catalytic converters, and therefore, zero lead emissions. The above assumption is made based on the fact that the 2000 AD emission standards are achievable only by using catalytic converters with unleaded petrol.

Actual emission from vehicles would depend on several factors such as age of the vehicles, vehicle condition, operating speed, etc. Though the operating speed will be more or less the same for all vehicles in a particular segment of the road, the other two factors i.e. age of vehicles and vehicle conditions would vary from vehicle to

vehicle. Therefore, these factors have not been considered in the emission estimation. Further, since it has been proposed to adopt emission standards prescribed by IIP for the year 2000 it is assumed that irrespective of vehicle condition and age of the vehicles, all the vehicles will have to conform to these standards.    The RTO will have to ensure that these emission standards are met by all the vehicles, through PUC implementation, which does not envisage different emission standards related to age or condition of the vehicles.

1.5.3	Stake Holders

Various stakeholders likely to be affected by vehicular pollution are : (i) population of the sub regions exposed to the vehicular pollution, (ii) people living along the major road links and (iii) commuters.      However, since traffic related pollution is usually confined to the road corridors, exposure to population living along the road corridors is used to indicate air quality impacts of the transportation alternatives.

Air quality impacts on the above group of people are estimated by assuming that only the people living within 150 meters on both the sides of the road segment are exposed to the pollution level estimated for the grid in which the road segments lies. The number of people living along the road corridors are estimated by using total road length, and population densities for the base year and future years in the four sub regions.

1.5.4	Air Quality Indices

Roll Back Model equations for the concerned pollutants, and their respective emissions computed for each grid is used to’ estimate pollutant concentrations in all the grids.    The estimated concentrations are used to determine quality index of each pollutant in each grid for the transportation options in the year 2011 with the help of appropriate value functions.    The overall quality index of each pollutant in a sub-region is then computed by using population along the road corridors as the weighting factor for averaging the' quality indices determined for each grid.    The overall quality index of a pollutant in the sub-region is defined as:

S Q I j Pj

I =

S Pj

Where,

Q I j      =    Quality Index of ith pollutant corresponding to its concentration Cij for grid j

Pj        = 	Population along the traffic corridors in grid j.

Ii         = 	Overall quality index of ith pollutant in the sub-region

The value functions of the pollutants are shown in Fig. 4.1.1 to 4.1.5.

1.6	NOISE IMPACTS

The operation of trains, cars, trucks, two wheelers and auto rickshaws also contribute to the generation of noise. The level of noise generated depends on a number of factors including type of vehicle, vehicle speed, number of vehicles (or traffic volume) and road condition. Since, different transportation alternatives would result in different vehicle composition in MMR, noise impacts would vary from one transportation alternative to another. The land use characteristics by virtue of attenuating noise propagation could also influence noise exposure to population.    For noise impact quantification and assessment, appropriate noise indicators/indices need to be identified.

The following steps are used to determine noise impacts of the transportation alternatives in the four sub regions and MMR as a whole.

- Identifying noise indicators and indices

- Estimating traffic composition and volume for road links of each sub region under each alternative using the Regional Transport Model

- Using an appropriate noise prediction model to estimate traffic related noise levels for the base year and the planning horizon years under the transportation alternatives

- Assessing traffic related noise impacts on different stake holders

1.6.1	Noise Level Indicators

Noise is undesired sound which gives rise to fluctuations in atmospheric pressure and causes an eardrum to vibrate.    In the middle ear, these vibrations are transmitted through a series of three small bones to the liquid filled inner ear.    The inner ear contains about 30,000 sensory cells, each with a small hair protruding in the liquid.    The liquid movement produces nerve impulses, which are carried by the auditory nerve to the brain where they are perceived as sound.

Noise induced hearing damage occurs in the inner ear due to two types of impacts - acoustic trauma resulting from sharp and very loud sound due to blowing apart of sensitive hair cells; and prolonged exposure to relatively lower noise levels resulting in the wearing of cell hairs. Noise standards are typically set up for protecting exposed population from the above two types of impacts.

Various noise parameters, which are of interest from noise generation and exposure view points, are Lio, Lso, Lgo and L«q.

L10 is the noise level that is exceeded 10 percent of the time, and generally represents peak noise level. This parameter is useful indicator for situations where short duration and high impulsive noise levels (e.g., 140 dBA) are expected.    Hence this indicator is not of much use to assess traffic related noise levels, which are expected to be in the range of 60-85 dBA.

L50 is the noise level that is exceeded 50 percent of the time and is close to the average noise level.

L90 is the level, which is exceeded 90 percent of the time and indicates background noise level.

Leq, or the equivalent sound level is an energy average of sound level during a specified period of time. From the noise exposure view point, Leq is a more meaningful parameter as it correlates well with the effects of noise on people. Hence, this indicator is used for the purpose of noise impact analysis.

The CPCB has established noise standards (Table 4.1.4) for protecting population likely to be exposed to different noise sources.    These noise standards are provided as Leq for Daytime and Nighttime.

Table 4.1.4    The CPCB Established Ambient Noise Standards

| Land Use | Leq dB(A) |  |
| --- | --- | --- |
|  | Day time (6.00 to 21.00 hrs) | Night time (21.00 to 6.00 hrs) |
| Industrial area | 75 | 70 |
| Commercial area | 65 | 55 |
| Residential area | 55 | 45 |
| Silence zone | 50 | 40 |

Traffic Noise Indicators

Noise generated from the transport activities needs to be assessed separately for day and night time since the intensity of its impacts is different during these times. Hence, noise levels predicted at 30 m from the centre line of the road are further processed to calculate the percentage length of links in a sub-region exceeding their day and night time standards respectively.    The percentage values so obtained are represented by Traffic Noise Index (TNI)" for day and night. Day time and night time TNI values are used to compare the noise impacts of the options or alternatives.

TNI (day) = % of length of road links in a sub-region exceeding day time standard. TNI (night) = % of length of road links in a sub-region exceeding night time standard.

1.6.2	Noise Modeling Analysis

Traffic Composition, Volume and Speed:

Traffic composition, volume and speed for each road link under different transportation alternatives are required for estimating traffic generated noise levels.      These traffic parameters are estimated    by    using the Strategic Transport Model of MMR.    This model is already discussed earlier in this section.

Noise Prediction Model:

The US Federal Highway Administration (FHWA) model is used for predicting noise levels at 30 m from the road central line due to each vehicle type (heavy and light) operating on a road link. The composite noise effect of heavy and light vehicles operating on the road link is obtained by the logarithmic addition of noise levels predicted for each vehicle type.    The model considers the effect of speed and noise absorption characteristics of the site on traffic generated noise levels.    Leq for heavy and light vehicles is calculated by using the following mathematical expression.

Leqi (h) = Loei + 10 Log (Ni/Si . T)+10 log (15/d) 1+ a + ds -13 where, Leqi (h) = Leq for hours 'h' for the i" vehicle type (heavy or light) Loei = reference mean sound level for the ith vehicle type Ni = number of vehicles of class 'i' passing during time T through a particular road section Si = average speed of the i" vehicle class in km/h in the particular road section T = duration for which Leq is desired and must correspond to time for which NI is measured. d = perpendicular distance in m from the centerline of the traffic lanes to the location of the observer. a = factor representing the adsorption characteristics of the ground cover between the roadway and the observer. ds = Shielding factor provided by noise barriers.

The reference mean sound levels for heavy and light vehicles are shown in Figure 4.1.6. For Island City, the noise absorption characteristics a is taken as 0.5 as the roads pass through high density built-up area. For other sub regions, a is taken as zero as major roads pass through open areas with a few exceptions. Other traffic parameters required to be input to the model are obtained from the Regional Transport Model.

The above noise prediction model is used to predict traffic noise for heavy and light vehicles for each road link at a distance of 30 m from the centre of the road. The predicted noise levels for heavy and light vehicles are then added together logarithmically

The modeling exercise is repeated for each link of the four sub regions for all the relevant transportation alternatives, and results are processed to calculate TNI values to facilitate the comparison of noise impacts across the sub regions and transportation alternatives.

FIGURE 4.1.6    REFERENCE MEAN SOUND LEVEL FOR HEAVY AND LIGHT VEHICLES

1.6.3    Stake Holders

Noise impacts due to traffic in an urban setting with high density of houses are highly localized, and mainly concentrated along road corridors.    Hence, commuters and people living along major traffic corridors are the two population groups likely to be impacted by traffic noise.    Traffic noise from the road / rail sections passing through green zones may also affect wildlife in these areas, but it is not considered in the analysis for lack of knowledge in this area.

The Traffic Noise Indices defined above are used to assess traffic generated noise impacts on population living adjacent to major roads.

1.7      ECOLOGICAL IMPACTS

Two types of ecological impacts, namely direct and development induced, are envisaged as a result of the implementation of rail and road projects under the specified transportation alternatives. Direct impacts result from the destruction of flora and fauna due to the construction of roads or railway lines. Since the introduction of roads and railway lines in a region is known to induce development in the area, hitherto virgin forests and wetlands could experience development pressure.    The severity of direct or development induced impacts on the ecological resources depends on the extent and richness of these resources at the road/rail project sites.    In the MMR region, though there are ecologically sensitive areas like mangroves, wetlands and forests, they are not unique in features and the threats posed by the transport and traffic related projects are not likely to cause any irreversible damage to the ecosystems.      Moreover, the proposed MUTP - II encounters very few ecologically sensitive areas and adequate mitigative measures can be provided to almost completely mitigate the adverse impacts.    There are no such rare and endangered species along the project sites, which once lost, will be lost forever, causing irreversible damage to the ecosystem. The degree of reversible impact caused by the transport and traffic related projects on the ecosystem would however vary from option to option. In view of the above, the ecological impacts of the alternatives are assessed at a qualitative level using the following framework.

- Description of location and existing ecological status of the ecologically sensitive areas through which the relevant road / rail link passes

- Qualitative description of direct and development induced impacts

- Mitigation measures to indicate the extent to which adverse ecological impacts could be alleviated.

1.7.1	Quantification of Ecological Impacts

In addition to providing qualitative analysis of ecological impacts, attempt is also made to quantify these impacts so that a comparison of these impacts could be made across the proposed transport alternatives. The rationale used for quantifying ecological impacts is as follows:

(i)	Damage to an ecological area due to a road/rail alignment will be proportional to    the    area    covered    by    road/rail    segment    in    the ecologically sensitive zone.

(ii)	Damage will be severe if the ecological area or system is of higher quality.

Further, because of the difference in the degree of importance attached to natural and man-made ecological systems, the impacts on these two types of systems are considered separately. The ecological impact on each of the system is measured by adding weighted areas of road/rail alignments traversing through the system, where weights represent the quality of the system.      Thus ecological impact indices for the two systems can be expressed as follows:

El (natural)        = EI1 = S Wni Ai

EI(man-made)	= EI2 = S Wmj Aj

Where,

Wni	=	weight associated with or importance given to a natural ecological

system of class i

Wmj	=                  weight associated with or importance given to a man-made ecological

system of class j

Ai	=	Total area of road/rail alignments intercepted by natural ecological

systems classified into category i.

Aj	=	Total area of road/rail alignment intercepted by man-made ecological

systems classified into j th category.

Classification of ecological systems and weights assigned to these classes or categories are provided in Tables 4.1.5 and 4.1.6 for natural and man-made systems, respectively. Higher value of the weight indicates higher quality of the ecological area, which is prone to more adverse impacts due to a road/rail project. Thus, higher values of EI represent potential for more adverse impacts on ecological systems.

The procedure described above for computing ecological impact indicators (EI1 and EI2) is used to quantify ecological impacts of the four transport options.    The results are presented in Section 4.

Table 4.1.5 Classification of Natural Ecosystems and Associated Weights

| Type of Area | Weight |
| --- | --- |
| Natural vegetation with high or moderate species diversity and tree canopy cover greater than 60% | 4 |
| Natural vegetation with high or moderate species diversity and tree canopy cover between 30 to 60% | 3 |
| Scattered trees with 60% ground cover of herbs, grasses and bushes (Species diversity moderate to low) | 2 |
| Ground cover of natural vegetation less than 60% or the degradation of the area is irreversible | 1 |

Table 4.1.6 Classification for Agricultural Land Plantations and Associated Weights

| Type of Area | Weight |
| --- | --- |
| Agricultural land with 2 crops or plantations with ground cover greater than 60% | 3 |
| Agricultural land with crops or plantations and ground cover between 30 to 60% | 2 |
| Fallow agricultural land or plantation with ground cover<30% | 1 |

  - SOCIAL IMPACT (LIMITED TO DISPLACEMENT OF PEOPLE)

The projects identified under various transport alternatives could give rise to wide range of social impacts as the implementation of some of these projects would require displacement of households and commercial establishments. The number of households and commercial establishments to be displaced provides the measure of adverse social impacts due to the proposed projects.

Hence,    for      each    sub    region    and    each      transportation      alternative,    number    of