APPENDIX 4.3

ENVIRONMENTAL ASSESSMENT OF FLYOVERS

1.1        INTRODUCTION

The Maharashtra State Road Development Corporation (MSRDC) along with Public Works Department (PWD) has undertaken the construction of flyovers on all major arteries and Highways with a view to eliminate the traffic congestion and delays at the busy intersections. Fifty locations have been selected for flyovers. The environmental assessment of flyovers was taken up with the objectives of :

•    Reviewing the current environmental status of the area along the flyover alignments

including identifying sensitive receptors,

•    Assessing direct, indirect, immediate and long-term environmental impacts of flyover

scheme,

•    Screening of flyovers to identify those which would improve ait quality,

•    Undertaking economic evaluation of environmental impacts, and

•    Preparing comprehensive environmental management plan.

The study focussed on the following six flyovers in Island city:

•    Dadar-Khodadad circle

•    S.V, Road - Barfiwala junction

•    Haji Ali - Wilson College

•    JJ Hospital-Crawford Market

•    Senapati Bapat (SB) Marg - Fergusson college Road

• SB Marg - Elphinstone

1.2	ENVIRONMENTAL STATUS

Environmental status for the above flyover locations was assessed with respect to ambient air quality, noise, water, land, biological and social components.

The concentrations of Sulphur Dioxide (S02), Nitrogen Oxides (Nox), Carbon Monoxide (CO), Respirable Particulate Matter (RPM), Total Particulate Matter (TPM) and Hydrocarbons (HC) were monitored at alt the locations. S02 and Nox concentrations were found in compliance with the standards at all locations. TPM and RPM exceeded standards at all the locations. He highest levels of TPM and ' Traffic, Economic and Environment Impact Assesiment of Flyovers in Mumbai - Task 3 Report, Tata Consultancy Services, 1994

No land acquisition is required for the construction of flyovers. However, at Haji Ali - Wilson College and JJ Hospital-Bhendi Bazar-Crawford Market location, roads are narrow and area is densely populated. Barricades at construction sites would cause visual obstruction.

Shifting and rearranging service lines below the road would affect the residents during shifting period.

Vibrations during piling and other construction activities may affect 15 heritage structures / precincts. The most susceptible is Gamdevi Precinct along Pandita Rambai Marg.

1.3	OPERATION PHASE IMPACTS

Flyovers will eliminate delay for through pass vehicles at intersections resulting in change in speed, fuel efficiency and emissions. It was found that CO, Nox and HC emissions would decrease only for one flyover (Dadar Khodadad Circle).

Due to splitting of traffic into two grades, emissions at higher elevations in open areas	 would lead to better diffusion.    However, at locations where flyovers are close to buildings, residents will be exposed to higher pollutant levels. This is particularly true for the sections of proposed flyovers at Pandita Rambai Road, Tardeo Road and Mohammed Ali Road.

Predicted concentrations of RPM are much lower than expected, which may be due to the exclusion of other RPM sources - wave breaking and area sources, etc.

Flyovers would increase traffic noise due to echo and reverberations. Peak hour road side noise at ground level was estimated to lie between 73 to 81 dB(A) for 1999 and 74.8 to 82.6 dB(A) for 2011.

1.4	ENVIRONMENT MANAGEMENT PLAN

1.4.1	Construction Phase

•    Movement of materials and workers to be restricted to non-peak hours

• DG sets and construction equipment, which generates air pollution and noise, not to be located near vulnerable receptors.

RPM were observed at Dadar- Khodadad Circle. One hour Co concentrations were found in compliance with the standards at all the locations, however, 8-hour CO concentration exceeded the standard at all the locations. Both CO and HC concentrations peaked around evenings.

High pollutant levels observed at Dadar - Khodadad Circle could be due to ongoing construction activities of flyovers at time when air quality was monitored.

Noise levels exceeded the standards at. all the locations. Noise levels monitored at the six locations are shown in Table 4.2.1.

Land use near flyovers is residential ?nd commercial. There are no terrestrial or aquatic ecosystems near these locations.

The above six flyovers are in the densely populated areas, having population density of 37033 persons / Km2, indicating high pressure on infrastructure. The heritage structures near flyover alignments are given in Table 4.2.2 and other sensitive receptors are given in Table 4.2.3.

1.4.2	Construction Phase Impacts

Part of the carriage way used for construction activities would reduce the road capacity and result in traffic congestion. The traffic congestion would give rise to higher vehicular emissions of CO, NOx, HC, and RPM.

The construction equipment would generate noise In the range of 108 to 132 dB(A). The noise impacts would be significant during night time, particularly, for nearby sensitive receptors.

The storm water runoff carrying loose soil from the construction site would not be significant except for monsoon season.

The labour force is not expected to reside at the flyover locations, and hence, environmental impacts associated with labour camps are not expected.

No direct change In land use is expected, as the flyovers would be constructed on the existing road alignments and the excavated material (if any) could be reused for construction purposes.

Some trees along flyover alignments need to be cut. The impact-is likely to be significant for Haji Ali - Wilson College stretch and JJ Hospital - Crawford Market stretch.

- Areas with loose and exposed soil to be sprayed with water to avoid fugitive dust emissions

- Trucks carrying material to the construction site to be covered with tarpaulin tp prevent fugitive    emissions

- Limiting construction noise to 90 dB(A) for eight hour work shift

- Use of hydraulic piling machines to reduce noise levels near sensitive receptors, such as, nursing homes.

- Workers to be provided with temporary housing with adequate civic facilities.

1.4.3	Operating Phase

- Strict emission norms for all types of vehicles

- Restricting    growth    of private    vehicles    in    Island    city through    demand management

- Providing efficient mass transportation facilities

- Encourage more CNG based vehicles

- Use environmental friendly alternative fuels

- Encourage road side plantation

- Educating vehicle drivers

- Plantation of creepers on both sides of flyovers

- Periodic monitoring of noise levels of vehicles to assess the compliance with Ministry of Environment and Forest (MoEF) prescribed standards

Table 4.2.1        Monitored noise levels in commercial and residential areas

|  | Noise Levels in dB(A)-Leg |  |  |  |
| --- | --- | --- | --- | --- |
| Location | Day |  | Night |  |
|  | Observed | Standard | Observed | Standard |
| Commercial Areas |  |  |  |  |
| Dadar Khodadad Circle | 69.5 | 65 | 63.3 | 55 |
| JJ Hospital-Bhendi Bazar-Crowford Market | 70.5 | 65 | 59.3 | 55 |
| Hutatma Chowk | 70.0 | 65 | 62.2 | 55 |
| Siddhivinayak Temple | 67.2 | 65 | 62.5 | 55 |
| Byculla Bridge | 71.0 | 65 | 59.5 | 55 |
| Residential Areas |  |  |  |  |
| SB Road-NM Joshi Marg | 65.8 | 55 | 58.2 | 45 |
| Haji Ali-Chowpatti | 69.0 | 55 | 62.5 | 45 |
| Barfiwala Junction | 69.8 | 55 | 60.6 | 45 |

Table 4.2.2            Heritage structures near flyover locations

| Proposed Flyover | Heritage Structure / Precinct |
| --- | --- |
|  | Haji Ali Complex |
|  | Mahalaxmi Precinct |
|  | Dhakleshwar Tennple |
|  | Ma Hajjani's Dargah Complex |
| Haji Ali-Wilson College | Old Milestone 2 |
|  | Adenwalla Mansion (Suresh Bhavan) |
|  | Wilson College Mackinnon Hostel |
|  | Anand Kannan (Villa with Temple & Wadi) |
|  | Gamdevi Police Station |
|  | Suhag Palace |
|  | JJ Hospital Complex |
|  | Old Parsi Ward |
|  | FD Petit Building |
|  | CircleGrant Medical College Building |
| JJ Hospital - Crawford Market | Crawford Market |
|  | Market Building |
|  | Fountains in the market |
|  | Minara Masjid |
|  | Fakhruddin Building |
| Dadar Khodadad | Khodadad Circle with buildings |

Table 4.2.3    Sensitive Receptors near the Proposed Flyovers

| Proposed Flyover | Heritage Structure / Precinct |
| --- | --- |
| Dadar Khodadad Circle | Mistry Maternity and Nursing Home |
|  | Shetty Nursing Home |
| Haji Ali-Wilson College | Vistria Memorial School for Blind |
|  | Bhatia School |
|  | Bhartiya Vidya Bhavan |
|  | Ajinkya Hospital |
|  | Wilson College |
|  | EEE Sassoon High School |
|  | Hume High School and Junior College |
|  | JJ Hospital |
|  | Haji Kareem Hohammad Suleman Maternity Home |
| JJ Hospital - Crawford Market | Ismail Beg Mohammad High School |
|  | Noor Hospital |
|  | Seth Husson Ali Abdul Ali Baseeruddin |
|  | Dholkawala Maternity Home |
|  | Mumbai Girls High School |
| SV Road-Barfiwala Junction | Sangeeta Maternity and Gynaecology Hospital |