APPENDIX 4.4

SECTORAL ENVIRONMENTAL IMPACTS OF FLYOVERS

  - GENERAL

Recently 50 flyovers have been planned in MMR, out of which 30 have been already constructed. Table 1 gives a list of all the flyovers.

The flyovers are expected to impact air quality and noise due to possible redistribution of traffic on various road links, shift of passenger trips from public transportation to private vehicles, and induced traffic due to facilitation of traffic flow by flyovers. Though, there would be inconvenience to public during construction stage, the flyovers would not disturb any fresh ecologically sensitive areas or cause major displacement of people. There may be need to cut few trees and temporarily displace few people, which constitute minor impacts. However, there could be social impacts.due.to visuahntrusion, higher-air pollution and noise exposure to people living in multi-storey.buildings close to flyovers, Therefore, environmental impacts of flyovers are considered with respect to air quality, noise and social aspects.

1.2	AIR QUALITY AND NOISE IMPACTS

The flyovers may cause change in air quality and noise impacts at sectoral level due to the following reasons.

(A) Redistribution of traffic on the road links: Due to perceived or actual increase in capacity due to flyovers, the flyover corridors may attract traffic from nearby road links, thereby giving rise to change in air and noise quality in these areas.

(B) Shift in passenger trips from public transportation to private vehicles: Again, due to perceived or actual increase in road capacity, people using public transport may start travelling by private cars and taxis. Hence, there could be increase in number of vehicles on roads resulting in increase of both air and noise pollution. .

(Conduced Traffic: Flyover corridors, by facilitating the flow of traffic, may give rise to induced traffic, which in turn would result in more noise and vehicular emissions

The air quality and noise impacts of flyovers are analyzed with respect to the above factors in the following paragraphs.

(A) Impacts due to redistribution of traffic:

Flyover may result in change of air quality impacts due to redistribution of traffic, as air quality impacts are related to traffic volume and traffic composition. A flyover corridor would result in redistribution of traffic, if the corridor is competing with other nearby road links for traffic.    The competing road links would generally be those links, which run parallel to a flyover corridor, as the destination of some of the commuters on these links will be in the direction of the flow of traffic on the flyover, corridor.    The redistribution of traffic will actually depend on the relative traffic congestion level on various competing road links.

Initially, flyovers may also attract traffic due to perceived increase in capacity of a "flyover corridor. However, ultimately traffic will adjust itself according to the real capacity of road links.

Table 4.5.1	 List of Flyovers

| Sr.No | Flyovers | Status |
| --- | --- | --- |
|  | Western Express Highway |  |
|  | Worli Bandra | done |
|  | Kherwadi | deferred |
|  | Kalina Vakola | executed |
|  | Santacruz-Airport | proposed |
|  | Janata Colony | NA |
|  | Jogeshwari-Vikholi Link Road | executed |
|  | Aarey Goregaon | executed |
|  | Goregaon Mulund Link Road | executed |
|  | Rani Sati Marg | executed |
|  | Shantaram Ralao | NA |
|  | Akurli Road | NA |
|  | Dattapada Road | executed |
|  | National Park | executed |
|  | Shiv Vallabh Road | executed |
|  | Sahar Glyover | Half built |
|  | Eastern Express Highway |  |
|  | Sion | executed |
|  | R.C.F. Junction | executed |

| Sr.No | Flyovers | Status |
| --- | --- | --- |
|  | C.S.T | NA |
|  | Chheda Nagar | NA |
|  | Andheri Ghatkopar Link Road | to be started |
|  | Vikholi Junction | under construction |
|  | Goregaon Mulund Link Road | executed |
|  | Nitin Casting | executed |
|  | Cadbury | executed |
|  | Golden Dyes | executed |
|  | Kapur Bawadi | executed |
|  | Sion Panvel Highway |  |
|  | B.A.R.C. | executed |
|  | Chembur Mankhurd Link Road | executed |
|  | Vashi | executed |
|  | Nerul Bhavan | executed |
|  | Uran Road | executed |
|  | Konkan Bhavan | executed |
|  | Kharghar | NA |
|  | Taloje | NA |
|  | Kamothe | NA |
|  | Mumbai City Roads |  |
|  | Haji Ali |  |
|  | JJ Hospital Bhendi Bazar | under construction |
|  | Crawford Market | under construction |
|  | SB Marg : N.M.Joshi | under construction |
|  | SB Marg : Ephinston | under construction |
|  | SB Marg : Ferguson | under construction |
|  | SB Marg : Connect toWEH | Not yet commenced |
|  | Adi Shankaracharya Marg LBS | under construction |
|  | Worli Naka | Not yet started |
|  | Siddhi Vinayak | deferred |
|  | Barfiwala Junction | NA |
|  | Adi Shankaracharya Marg : JVLR | under construction |
|  | Dadar Khodadad Circule | Completed |
|  | Juhu Airport Subway | Not yet Planned |

Appendix 4.3 presents qualitative analysis of flyovers with respect to their potential for the redistribution of traffic and resulting environmental impacts. The highlights of the analysis are provided as follows:

- The flyovers may marginally increase the vehicle carrying capacity of the highways by facilitating smooth traffic flow.    As a result, environmental quality on the stretches, which are not competing for traffic with nearby road links, would improve. However, there would be no environmental benefits from flyovers for the stretches competing for traffic with neighboring road links, as traffic redistribution would take place according to relative congestion levels. Further, with the growth of population and traffic, this marginal benefit of increased carrying capacity due to flyovers may dissipate over time. Most of the flyovers do not increase physical carrying capacity of the roads, as these merely lift the central lanes to a higher level.

- The flyovers do allow smooth traffic flow on only one of the intersection links. Therefore, there would be local environmental benefits as the vehicle idling time and resulting pollution at the intersection would reduce. However, if the flyover reduces ground grade capacity of the road for turning traffic, there could be long vehicle queues resulting in even greater pollution at local level.

- A number of flyovers, particularly on Western Express Highway, are constructed on minor cross-link roads. At local level, such flyovers would result in marginal environmental benefits. However, these small environmental benefits may not justify the cost of flyovers. Some of these flyovers are at the distance of less than 1.5 Kilometres.

- The flyovers located in high population and high road density areas, such as Island City, may result in redistribution of traffic. However, due to high road density, the overall traffic related environmental impacts in the area would remain the same.

- The redistribution of traffic and hence the associated change in air and noise quality impacts would take place on some stretches of WEH and EEH due to proximity of the competing traffic roads -the SV Road and the LBS Marg, respectively.

- In the congested areas of Island City and Suburbs, the proximity of flyovers to residential flats at higher storeys would expose the residents to higher levels of vehicular pollution and noise,, besides disturbing their privacy..

- The flyovers being mainly concentrated on two north-south aligned highways, would encourage more traffic flow into and out of Island City.

(B) Impacts due to shift from public to private transport

The TCS flyover study (1999) indicates that there would be shift of passenger trips from public transportation to private vehicles due to flyovers in Greater Bombay. The shift during peak hours is estimated to be about 3.2% in 1999 and 1.3 % in 2011. The shift would obviously increase cars and taxis on Mumbai roads. However, the number of buses or trains are not likely to get reduced due to this shift of passenger trips. At the most crowding in buses and trains might reduce insignificantly. Therefore, the overall number of vehicles on the roads would increase, resulting in increase in air and noise pollution levels. However, 3.2 % shift of passenger trips in 1999 would marginally increase pollution levels.    In    2011, increase in pollution levels due to shift of passenger trips (1.3%) would be even smaller.

(C) Impacts due to Induced Traffic

The detailed flyover study also indicates that the flyovers would give rise to induced traffic, and hence increase in environmental impacts. Flyovers, by facilitating vehicular flow, might encourage more people to travel, who hitherto avoided travelling because of traffic congestion. In other words, induced traffic would reflect hidden travel demand. This demand needs to be addressed rather than suppressed, otherwise the purpose of transportation service would be lost.

The above discussion was based on the qualitative analysis of impacts due to flyovers. Though from the qualitative discussion, it appears that the air quality and noise impacts due to flyovers would be marginal, it is desirable to substantiate this claim quantitatively. Since, it was obvious from the qualitative analysis that the change in impacts would be realized near flyover corridors and the competing road links only, the four corridors considered for analysis were: Western Express Highway (WEH), Swami Vivekananda Road (SV Road), Eastern Express Highway (EEH) and Lal Bahadur Shastri Marg (LBS). No estimate has been made for change in environmental quality in Island City due to flyovers, as the vehicular in-flow to city, and hence resulting environmental impacts, could be controlled by demand management measures. The changes in air and noise quality due to flyovers on the above four corridors were estimated by using the following procedures.

i.                    Air Quality

The most simple representation of an air quality model, which was used to estimate incremental ambient pollutant concentration (c) due pollutant emissions (Q) is as follows:

C=K.Q

Where K is a constant depending upon dispersion characteristics of the site.

Using this expression, the change in air quality impacts due to flyovers was obtained by

Change in air quality impacts = (Q2-Q1)/Q1

Where Q) and Q; represented emissions without and with flyovers.

Further, since emissions depend on vehicle type. Q1 and Q2 were estimated using the following expressions.

Q1 = (~n,.e1). Vehicular density without flyover

Q2 = (~n„ e1). Vehicular density with flyover

Where ni represents fraction of vehicle type i. and e, emission factor for thal vehicle type For estimating Q2, shift from public to private vehicles was accounted by increasing fraction of cars and taxis by 3.2 % for 1999. Vehicular density on roads was estimated using speed data from the TCS study, and speed curves from WS Atkins study. The change in air quality impacts due to flyovers was estimated with respect to CO, NOx and 50, for four corridors WEH. SV Road, EEH and LBS Marg for 1999 (Table 2). Except for Kurla-Sion Stretch of EEH, change in pollutant concentration levels ranged from .1 to 10 percent, which may not be considered as significant. However, the Kurla-Sion Stretch of EEH showed an increase by about 85 percent in pollutant concentrations due to flyovers.

Table 4.5.2 Estimated percent changes in air quality impacts

| Road Link | PCU/Km NFL | PC U/Km WFL | % change CO | % change SO2 | % change NOx |
| --- | --- | --- | --- | --- | --- |
| Dahisar-Jogeshwari (WEH) | 868 | 805 | -6 | -6 | -7 |
| Jogeshwari-Bandra (WEH) | 616 | 627 | +10 | +10 | +8 |
| SV Road | 869 | 847 | -1 | -1 | -2 |
| Thane-Vikhroli (EEH) | 977 | 1757 | NC | NC | NC |
| Vikhorli-Sion (EEH) | 688 | 654 | +84 | +85 | +84 |
| LBS Marg | 574 | 561 | -3 | -2 | -2 |

- Noise Quality

The change in noise quality impacts due to flyovers was estimated by using FHWA model represented by the following equation.

Leqi (h) = Loei + 10 Log (Ni/Si . T)+10 log (15/d) 1+ a + ds -13 where, Leqi (h) = Leq for hours 'h' for the i" vehicle type (heavy or light) Loei = reference mean sound level for the ith vehicle type Ni = number of vehicles of class 'i' passing during time T through a particular road section Si = average speed of the i" vehicle class in km/h in the particular road section T = duration for which Leq is desired and must correspond to time for which NI is measured. d = perpendicular distance in m from the centerline of the t raffic lanes to the location of the observer. a = factor representing the adsorption characteristics of the ground cover between the roadway and the observer. ds = Shielding factor provided by noise barriers.

Assuming there was no absorption (a = 0) and using reference sound level for 20 kms /hr speed, the above equation was simplified to estimate Leq for heavy and light vehicles.

Leq (h) = 67.62 + 10 log [Ni/St.T]

Leq (1) = 47.6 + 10 log (Ni/Si.T]

Using PCU flows and speeds for heavy and light traffic for flyover and without flyover cases, change in noise levels due to flyovers was estimated to be in the range of 1dB(A) to 3 db(A).

Except for one road link (Vikhroli - Sion), quantitative analysis has shown marginal change in air quality due to flyovers. Even if noise level by 3 dBA with flyovers could be of concern, particularly if the traffic noise levels are already high. Further, there are a number of impacts brought forth by qualitative analysis for which mitigation measures must be identified. These mitigation measures are discussed in the subsequent sections.

iii.                Social Impacts

- Flyover, would be too close to some residential flats in densely populated areas. The people living in these areas would experience visual intrusions and higher- noise and pollution levels.

- TCS study has identified many heritage structures and sensitive receptor near the flyovers in Island city. These structures and receptors are listed in Appendix 5.1. Pile driving operations during construction stage may cause damage to heritage structure.

- The property prices will come down near flyovers.

- There is one settlement along SB Marg - MM Joshi Marg alignment which would be disturbed during construction.

  - MITIGATION OF AIR QUALITY IMPACTS AND NOISE LEVEL

- Traffic congestion due to turning traffic at flyover intersections and resulting high pollution levels could be mitigated by widening of ground grade lanes, and thereby, reducing congestion and pollution levels at these intersections.

- To overcome the negative effect of traffic bottlenecks on traffic flow the width of flyover corridors (particularly Highways) should be made uniform.

- Perceived or actual increasing traffic carrying capacity of flyover corridors would attract traffic from the neighbouring competing roads, and thereby, give rise to traffic congestion and higher levels of pollution. It may be noted here that the redistribution of traffic would depend on the relative congestion level of the competing road links. Flyover corridors would attract traffic, only if the competing road links do not have sufficient capacity to carry traffic. In fact, this would indicate that there is need for creating new transportation links to meet the travel demand.

- In Greater Bombay, flyovers are mainly concentrated on WEH and EEH in suburbs. Both the highways run almost parallel to Western and Central railway lines.    Therefore, by increasing the passenger carrying capacities of both these railways, the congestion and pollution levels on the highways could be reduced

- The flyovers are also planned to be on East-West links connecting two highways The traffic congestion on these links need not to be linked to flyovers, but may be due to the fact there are only few such links to address the demand for east west moverne'll    Traffic.

- Increasing road and rail capacity to accommodate more traffic and reduce

congestion on the flyover corridors.

- Shifting of bus stops to road shoulders to free the left lane.

Traffic management measures:

- Dedicating left lane exclusively for buses.

- Introducing demand management measures to discourage shift from public transport to private vehicles.

- Encouraging car pooling and telecommuting

Design level measures:

- Developing guidelines for the location of flyovers, which would take environmental aspects into consideration.

- Developing guidelines for the design of flyovers based on the aim of requirements of turning and through traffic.

Social:

As already discussed above.

1.	Generic mitigation measures identified for the transportation    sector as a whole would reduce the environmental impacts of the flyovers as well. With the flyovers in MMR, more thrust needs to be placed on encouraging the use of public transport, modifying travel demand in the island city by demand management measures and improving traffic circulation in Island City.

2.	Though the flyovers are not part of IVIUTP, the investment schemes of MUTP are designed to encourage public transport and increase traffic circulation in congested areas. Thus. MUTP remains the valid transport choice even with flyovers.

3. 	Mitigation for alleviating environmental impacts of individual transport projects are presented in the next section. The section also discusses the environmental impacts of MUTP investment schemes.