APPENDIX    7.1

National Ambient Air Quality Standards

| Pollutants | Time Weighted Average | Concentration in ambient air |  |  | Method of measurement |
| --- | --- | --- | --- | --- | --- |
|  |  | Industrial Area | Residential Rural & other areas | Sensitive Area |  |
| Sulphur Dioxide(SO2) | Annual Average* | 80 mg/m3 | 60 mg/m3 | 15 mg/m3 | - Improved West and Geake Method - Ultraviolet Fluorescence |
|  | 24 hours** | 120 mg/m3 | 80 mg/m3 | 30 mg/m3 |  |
| Oxides of Nitrogen as NO2 | Annual* | 80 mg/m3 | 60 mg/m3 | 15 mg/m3 | - Jacob & Hochheiser Modified (Na-Arsenite) Method - Gas Phase Chemiluminescence |
|  | 24 hours** | 120 mg/m3 | 80 mg/m3 | 30 mg/m3 |  |
| Suspended Particulate Matter (SPM) | Annual* | 350 mg/m3 | 140 mg/m3 | 70 mg/m3 | High Volume Sampling, (Average flow rate not less than 1.1 m3/minute) |
|  | 24 hours** | 500 mg/m3 | 200 mg/m3 | 100 mg/m3 |  |
| Respirable Particulate Matter (PM10) , size less than 10 mm) | Annual* | 120 mg/m3 | 60 mg/m3 | 50 mg/m3 | Respirable particulate matter sampler |
|  | 24 hours** | 150 mg/m3 | 100 mg/m3 | 75 mg/m3 |  |
| Lead (Pb) | Annual* | 1.0 mg/m3 | 0.75 mg/m3 | 0.50 mg/m3 | ASS Method after sampling using EPM 2000 or equivalent Filter paper |
|  | 24 hours** | 1.5 mg/m3 | 1.00 mg/m3 | 0.75 mg/m3 |  |
| Carbon Monoxide (CO) | Annual* | 5.0 mg/m3 | 2.0 mg/m3 | 1.0 mg/m3 | Non dispersive infra red Spectroscopy |
|  | 24 hours** | 10.0 mg/m3 | 4.0 mg/m3 | 2.0 mg/m3 |  |
| Ammonia | Annual* | 100 mg/m3 | 100 mg/m3 | 100 mg/m3 | -- |
|  | 24 hours** | 400 mg/m3 | 400 mg/m3 | 400 mg/m3 |  |

*	Annual Arithmetic mean of minimum 104 measurements in a year taken twice a week 24 hourly at uniform interval.

** 24 hourly/ 8 hourly values should be met 98% of the time in a year. However, 2% of the time, it may exceed but on two consecutive days.

NOTE:

- National Ambient Air Standard: The levels of air quality with an adequate margin of safety, to protect the public health, vegetation and property.

- Whenever and wherever two consecutive values exceeds the limit specified above for the respective category, it would be considered adequate reason to institute regular/continuous monitoring and further investigations.