ENVIRONMENTAL MANAGEMENT PLAN

## 7.1	INTRODUCTION

EA Process

In response to the deteriorating transport scenario in MMR, the MMRDA prepared a Comprehensive Transport Plan in 1994, which provided a strategic framework for improving the transport sector of MMR.  The four strategic transport options identified under this study were subjected to Environmental Assessment (EA) in the form of Sectoral Level Environmental Assessment (SLEA), to identify the one, which would have minimum adverse environmental impacts.  This resulted in a preferred strategy comprising a larger set of projects. Considering the financial sustainability and the economic evaluation of subprojects an investment program comprising a subset of projects recommended in the preferred sectoral strategy is arrived at.  This subset of projects is then subjected to Programmatic Level Environmental Assessment (PLEA) and Micro-Level Environmental Assessment (MLEA).  A flowsheet of the entire EA process is given in Figure 7.1.

Apart from the sectoral environmental impacts, the individual projects will have localized environmental impacts.  The sub-projects included pertain to rail and road improvement schemes. The projects were screened with respect to various environmental parameters such as air and noise pollution, sensitive locations etc., to determine the type of EA required.   PLEAs were prepared for small generic sub projects, such as, flyovers, ROBs, Pedestrian subways, Station Area Traffic Improvements Schemes (SATIS). MLEAs were done for individual projects that were expected to have significant environmental impacts.  The type of EA for each of the project in MUTP is given below in Table7.1.

Table 7.1  List of Projects Selected for PLEA/ MLEA

| Sr.No. | Name of the Project | Type of EA | Status of EA/EMP EMP Preparation | Schedule of Completion |
| --- | --- | --- | --- | --- |
| Rail Component |  |  |  |  |
| 1 | 5th line between Santacruz and Borivali | EMP | These works are in an advanced stage of completion. Environmental audit and EMP are to be completed. | 31/1/02 |
| 2 | 5th and 6th lines between Kurla and Thane | EMP | These works are in an advanced stage of completion. Environmental audit and EMP are to be completed. | 31/1/02 |
| 3 | Borivali-Virar Quadrupling of Lines - Borivali Bhayander Rail line - Bhayander – Virar Rail line | MLEA | EA & EMP done in 1998. EMP being updated. | 31/1/02 |

Table 7.1  Cont.

| Sr.No. | Name of the Project | Type of EA | Status of EA/EMP EMP Preparation | Schedule of Completion |
| --- | --- | --- | --- | --- |
| 4 | Optimization on western Railway (including 12-car rakes on through lines) | EMP/ CEMP | PAH have been resettled in permanent / transit accommodation. CEMPs are being prepared for these resettlement sites. | 31/1/02 |
| 5 | Optimisation on Central Railway (including 12-Car rakes on through lines) | EMP/ CEMP | “ | “ |
| 6 | Optimisation on Harbour line | EMP/ CEMP | “ | “ |
| 7 | DC/AC conversion | EMP | Being Prepared | 31/1/02 |
| 8 | EMU coach re-manufacturing | No EA Req. | -- | -- |
| 9 | Track machines | No EA Req. | -- | -- |
| 10 | Technical Assistance | No EA Req. | -- | -- |
| Roads and Traffic Component |  |  |  |  |
| 1 | Jogeshwari-Vikhroli Link Road | MLEA | EA & EMP done in 1998. EMP being updated. | 15/12/01 |
| 2 | Santacruz-Chembur link road | MLEA | EA / EMP Being prepared as a part of the engineering feasibility study. Independent Review of EMPs | 31/12/01 31/1/02 |
| 3 | ROB at Jogeshwari (South) | EMP | Being done on the basis of PLEA for ROB at Vikhroli. | 31/1/02 |
| 4 | ROB at Jogeshwari (North) | EMP | Being done on the basis of PLEA for ROB at Vikhroli. | 31/1/02 |
| 5 | ROB at Vikhroli | PLEA | EA & EMP done in 1998. EMP being updated. | 31/1/02 |
| 6 | Dadar - Mahim One way system | No EA Req. | -- | -- |
| 7 | Pedestrian subways and bridges | EMP | EA & EMP for CST (VT) done in 1998. EMP for other subways being done on basis of CST- EA / EMP. Independent Review of EMPs | 31/3/02 30/4/02 |
| 8 | Station area traffic improvement schemes (SATIS) | EMP | EA/EMP for Thane done in 1998. EMP for other SATIS being done on the basis of Thane EA / EMP. Independent Review of EMPs | 31/3/02 30/4/02 |
| 9 | Other traffic management schemes | No EA Req. | -- | -- |
| 10 | Procurement of Buses | No EA Req. | -- | -- |

Table 7.1  Cont.

| Sr.No. | Name of the Project | Type of EA | Status of EA/EMP EMP Preparation | Schedule of Completion |
| --- | --- | --- | --- | --- |
| 11 | Environment-air quality monitoring | No EA Req. | -- | -- |
| 12 | Technical Assistance, studies training | No EA Req. | -- | -- |

Projects at 8 to 10 of Rail do not involve any civil work.  EA is thus not required.

Progress at 9 to 12 of Road Component are related to traffic and air quality management and does not involve any Civil work. EA is thus not required.

FIGURE 7.1	ENVIRONMENTAL ASSESSMENT PROCESS

Comprehensive Transportation Study

4 strategic options

- Do minimum

- Public transport option

3. 	Public transport + Demand management

4. 	Road Investment option

1994

1996

1997

1998

2001

2001

Sectoral Level Environmental Assessment (SLEA)

Evaluation of strategic options with respect to the following environmental criteria

- Traffic Analysis

- Impact on Air, noise, social, ecological  resources and transportation service quality

Screening of projects with respect to potential impacts

Micro-level Environmental Assessment (MLEA)

- Subprojects selected – road improvements, suburban railway subprojects

- Impact on traffic, air, noise, water

landuse, social and ecological

resources.

Programmatic Level Environmental Assessment (PLEA)

- Sub-projects selected: ROBs, pedestrians subways

- Analysis of traffic & transport

- Impact on air, noise, social and ecological resources, landuse

Selection of strategic option (PT + DM)

Preparation of updated and consolidated EA and project specific EMPs

# Draft Consolidated EA

Public Consultation

Final Consolidated EA

## Note: The current project stage is highlighted In the above Figure

Public Consultation

Environmental impacts of each of the above projects are described in Chapter 6 and a summary of impacts is given in the following Sections.

Environmental Management Plans

Plans are prepared for mitigating and managing potential adverse environmental impacts identified in SLEA and project specific PLEA/MLEA studies.   These EMPs are accompanied by institutional and reporting framework to ensure EMP implementation and monitoring programs for individual projects.

## 7.2 	Sectoral level EnvironMental mitigation and Management plan

In spite of having a transport strategy with the least environmental implications for the region, MMR is bound to experience environmental impacts from transportation sector on account of increase in both human and vehicle population.  There are several environmental issues that need to be addressed at the sectoral level e.g. increase in air and noise pollution, ecological impacts from extension of railways in ecologically sensitive areas, rehabilitation of displaced people etc. Mitigation measures for addressing the above issues are taken up at sectoral level using the SLEA framework.

Reducing traffic congestion, vehicular emissions and noise could alleviate the impacts due to the development of the transportation sector as a whole. The mitigation measures are developed at legal, policy and administrative levels for reducing sectoral level environmental impacts.

7.2.1	Sectoral Level Environmental Impacts

The SLEA was carried out for the four suggested transport options with respect to six major parameters viz. Transport and Traffic Efficiency, Ambient Air Quality, Noise, Ecology and Social (R & R).  Impacts on these parameters were studied in four major sub-regions of MMR viz. the Island City, Eastern Suburbs and Western Suburbs of Greater Mumbai City and the Rest of MMR (See Map in Figure 1.1 in Chapter 1).

In order to assess the impacts, a number of indicators/indices were identified for each of the above environmental parameters.  Baseline monitoring of the parameters were carried out in the year 1996 and the impacts were predicted for all the transport options for the year 2011.  These are presented in detail in Chapter 4.  The critical environmental issues that need mitigation measures at the sectoral level are summarised below:

- Ambient air quality (mainly for PM10 ) would deteriorate and noise levels would increase due to project implementation.

- Displacement of project affected households.

- Ecological impacts.

- Traffic Management and Safety.

7.2.2	Sectoral EMP

At the sectoral level, a variety of policy, legal and administrative measures could be used to manage environmental impacts. These are described in Table 7.2 (a) .

The formulation and implementation of several of the measures indicated depend upon a variety of parameters such as promulgation of rules and standards particularly by GOI, techno-economic feasibility of the suggested measures, public pressure, directives of Supreme Court and High Court etc.  Thus the time frame for implementation of these measures cannot be decided as a part of MUTP by the project authorities or GOM alone.  However, a suggestive framework for implementation of the EMPs including institutional responsibility, authorities and time frame is given in Table 7.2 (b).

7.2.3   Sectoral Level Monitoring

Sectoral level monitoring is needed to assess the success of the mitigative measures in improving the environmental quality and arresting the deterioration of the environmental quality and also to update EMP based on the feed back obtained from monitoring.

The monitoring plan presented here mainly includes monitoring of the environmental quality, R&R  and monitoring traffic parameters.  The plan is given in Table 7.3

### Monitoring of Environmental Quality

Environmental quality particularly with respect to air pollution and noise levels, must be monitored on continuous basis, as these environmental components are likely to be affected most by the transportation strategy.

Environmental quality monitoring shall be carried out with the objective of assessing the change in environmental quality due to the transportation strategy, so that the strategy can be updated in response to the monitored results.  Since the environmental quality of Greater Mumbai is also affected by factors other than transportation activities, environmental monitoring programme for the transportation strategy shall be integrated with the regional environmental monitoring programme. The recommended minimum environmental quality-monitoring programme with the focus on the transportation strategy is given in Table 7.3. The location of monitoring sites shall be decided by MMRDA in consultation with MCGM.

Monitoring of Social (R&R) Components

A number of MUTP projects will result in displacement of population.  The resettlement and rehabilitation process of the displaced population will be closely monitored by MMRDA and IMP. The methodology for such monitoring is given in paragraphs 64 to 68 of Resettlement Action Plan, September 2001, MMRDA.

Monitoring of Traffic Movement

The MUTP project is expected to bring about a substantial improvement in traffic movement in MMR and Greater Mumbai.  Various traffic  parameters such as volume, speed etc. for roads and  passenger trips for railway will be monitored to track changes in traffic movement.  As a part of MUTP, a Traffic Management Unit (TMU) is being established in MCGM.  The TMU will undertake periodic studies to monitor the traffic movements and impact of proposed measures in MUTP.

Table 7.2 (a)  Sectoral EMP

| Environmental Impact | Regulatory Measures | Policy Measures | Administrative Measures |
| --- | --- | --- | --- |
| Ambient Air Quality Deterioration | - Stringent emission standards for vehicles at manufacturing and operational level, - Use of cleaner technologies, such as, catalytic converters for petrol driven vehicles, four stroke engines for two and three wheelers, - Measures to prevent fuel adulteration, - Legal support for policy and administrative level mitigation, - Permitting use of alternate fuels such as CNG and LPG as automobile fuel. (Already done as per Liquefied Petroleum Gas (Regulation of Use in Motor Vehicles) Order, 2001. | Review and Implementation of recommendations made by committee for reducing vehicular pollution in Mumbai, constituted under the Mumbai High Court Order dated December 15, 1999. Some of the major recommendations are: - Reducing sulfur content of diesel initially to 0.05% and finally to 0.005% by 1st April 2005. - Reduction in Benzene content of petrol from the present level of 3% to less than 1%. - Refineries are recommended to implement the `Marker’ system for detecting adulteration in fuels and lubricants. - All the retail outlets in MMR should sell only pre-mixed petrol through dispensers to two and three wheelers with effect from 1st October 2000 - All petrol taxis above the age of 15 years and diesel taxis above the age of 8 years be converted to CNG or any other clean fuel and all diesel taxis above the age of 8 years should be converted to clean fuel3 wheelers above the age of 10 years to be scrapped or converted on CNG or any other clean fuel. All transport vehicles, except 3 wheelers and BEST buses over the age of 15 years to be scrapped unless converted to clean fuel. All BEST buses older than 20 years to be scrapped or converted to CNG. - All petrol driven vehicles registered in Mumbai prior to 1st April 1995 are recommended to be fitted with catalytic converters. . (Court is yet to decide) | - Strict enforcement of air emission - Enhancing of the current monitoring network of MCGM as per recommendation given in the study “ Strengthening the Air Quality Monitoring Network Operated by MCGM” completed in year 2000. - To develop PM10 abatement strategy. (Study commenced by MMRDA in Sept. 2001) - Vehicle Inspection and Monitoring Program : Promote vehicle inspection and maintenance centres. Commercial vehicles to obtain a Certificate of Fitness from approved Inspection and Maintenance Centres. |

Table 7.2 (a) Cont..

| Environmental Impact | Regulatory Measures | Policy Measures | Administrative Measures |
| --- | --- | --- | --- |
| Noise | -- | Explicit policy for noise reduction from vehicle engines, exhaust, tyre and road surface. | Monitoring of noise levels at sensitive receptors. Identifying sources of noise. |
|  |  | Reducing traffic noise impact by placing noise barriers to the extent feasible. |  |
| Social Impact | Government of Maharashtra adopted a policy of R & R in 1997. All resettlement to be undertaken according to the policy. | Project affected households to be rehabilitated as per the Rehabilitation Action Plan (RAP) and project specific RIP, CEMP. More details in Chapter 8. | PMU - MMRDA is responsible for entire R& R. An independent Monitoring Panel has been appointed to monitor the same. |
| Ecological Impacts | MMR – Regional Plan provides land use zoning and development control regulations consistent with these regulations. GoM has prepared Coastal Zone Management Plan (CZMP), which has been approved by MoEF. | The Indian Forest Act and Coastal Regulation Zone (CRZ) notification of 19/2/91 provide necessary policy framework to minimise ecological impacts. | Strict enforcement of Development regulations to contain growth in ecologically fragile areas. |
| Traffic Management and Road Safety | -- | Implementing recommendations of the committee appointed by Mumbai High Court (See above). The major traffic management policy recommendations include: - Strengthening traffic and transportation planning, traffic operation, traffic signals and signs, and area traffic control. - Modal integration outside railway station and at ferry landings, control of on-street and off-street parking, management of traffic demand, power to collect certain parking fees and supplementary charges on road traffic. | - Enforcement of road safety rules. - Development of emergency plans to deal with road/ rail accidents involving hazardous substances. |

Table 7.2 (b)  Sectoral EMP Implementation Framework

| Parameter | Mitigation Measures | Responsibility | Time Frame | Supervision |
| --- | --- | --- | --- | --- |
| # AIR POLLUTION |  |  |  |  |
| Regulatory Measures | Stringent emission standards, Use of Clean Fuel etc. | Govt. of India | Already in progress. | Not Applicable. |
| Policy Measures: | Several policy measures have been suggested for use of clean fuel, scrapping of obsolete vehicles etc. Several of such measures are pending clearance from Mumbai High Court. | GOI / GOM | Policies are in various stages of being drawn up. e.g Mumbai High Court decision of October 2001 requires vehicles more than 15 years to be scrapped or convert to CNG. | Various agencies as and when policies are implemented. |
| Administrative Measures | Strict enforcement of vehicular air emission standards. | RTO | Already in progress | GOM |
|  | Enhancing of the current air quality monitoring network of MCGM as per recommendation given in the study “ Strengthening the Air Quality Monitoring Network Operated by MCGM” completed in year 2000. | MCGM | To commence along with MUTP project implementation. | MMRDA |
|  | To develop PM10 abatement strategy. | Various agencies identified in the on-going study. | Study commenced by MMRDA in Sept. 2001. | MMRDA |
|  | Vehicle Inspection and Monitoring Program. | RTO | Plans to be drawn up and implemented during MUTP. | MMRDA |

Table 7.2 (b) Continued

| Parameter | Mitigation Measures | Responsibility | Suggested Time Frame | Supervision |
| --- | --- | --- | --- | --- |
| NOISE |  |  |  |  |
| Policy Measures | Explicit policy for noise reduction from vehicle engines, exhaust, tyre and road surface. | RTO | Fixing of time frame is not possible at this point. | MMRDA |
|  | Reducing traffic noise impact by placing noise barriers to the extent feasible. | RTO |  | MMRDA |
| Administrative Measures | Monitoring of noise levels at sensitive receptors. Identifying sources of noise. | MCGB & RTO | Within 1 year | MMRDA |
| SOCIAL IMPACT |  |  |  |  |
| Policy Measures | PIA - MMRDA is responsible for entire R& R. An independent Monitoring Panel has been appointed to monitor the same | GOM | Already done | -- |
| Administrative Measures | Project affected households to be rehabilitated as per the Rehabilitation Action Plan (RAP) and project specific RIP, CEMP. More details in Resettlement Action Plan, September 2001. | PIA | To be done before construction | MMRDA |
| ## Ecological Impacts |  |  |  |  |
| Administrative Measures | Strict enforcement of Development regulations to contain growth in ecologically fragile areas. | UD | Already being done | MMRDA |
| ## Traffic Management and Road Safety |  |  |  |  |
| Policy | Strengthening traffic and transportation planning, traffic operation, traffic signals and signs, and area traffic control. | RTO, MCGM – TMU | In Progress – To be strengthened in MUTP. | MMRDA |
| Administrative | Modal integration outside railway station, control of on-street and off-street parking, management of traffic demand, power to collect certain parking fees and supplementary charges on road traffic. | RTO, MCGM-TMU | To be established during MUTP. | MMRDA |

Table 7.3  Environmental Quality Monitoring

| Parameters to be monitored | Recommended Sites | Sampling duration and Frequency | Responsibility |
| --- | --- | --- | --- |
| Ambient Air Quality : |  |  |  |
| CO, HC, Nox, SPM, PM10, SO2 | As per recommendation given in the study “ Strengthening the Air Quality Monitoring Network Operated by MCGM” completed in year 2000. | As per recommendation given in the study “ Strengthening the Air Quality Monitoring Network Operated by MCGM” completed in year 2000. | MCGB |
| Noise : |  |  |  |
| To compute Lequ, L10, L50, and L90 | Same sites as above | Same as Above |  |
| Social (R&R) |  |  |  |
| Living Conditions of PAH’s. Drinking water supply, sanitation and other facilities | At resettlement site | 1 year after resettlement. | MMRDA |
| Traffic Movement |  |  |  |
| Traffic volume, speed | At individual sub-project sites and main arterial routes | At least once in a year | RTO/ MCGB-TMU |

## PROJECT level EnvironMental mitigation and Management plan

As discussed earlier, individual projects would give rise to localised environmental and social impacts.  The environmental impacts were assessed at two                levels - (a) Micro Level (MLEA) for individual projects that were expected to have significant environmental impacts and (b) Programmatic level (PLEA) for small generic sub projects, such as, flyovers, ROBs, Pedestrian subways, Station Area Traffic Improvements Schemes (SATIS).  The projects, which have been finally selected for implementation and for which EMPs have been prepared, are given earlier in Table 7.1.

7.3.1	Summary of Environmental Impacts/Issues

As seen in Table 7.1 given earlier, EA and EMP for the following projects have been  prepared in the year 1998. The environmental impacts of these projects and related aspects considered for mitigation plans are described in Table 7.4 (a) to (e) respectively attached at the end of this Chapter:

MLEA

- Jogeshwari-Vikhroli Link Road (JVLR)

- Borivali-Virar Quadrupling of Rail Lines(BVQR)

PLEA

- Road Over Bridge at Vikhroli

- Pedestrian Subway at CST

- Station Area Improvement at Thane Station

    - Environmental Mitigation Measures and Management Plans

The status of EA/EMP preparation is described in Table 7.1 earlier.  This section details the environmental management plans or mitigation measures (EMPs) developed for the following 5 sub-projects:

- Jogeshwari-Vikhroli Link Road (JVLR)

- Borivali-Virar Quadrupling of Rail Lines (BVQR)

- Road Over Bridge (ROB) at Vikhroli

- Pedestrian Subway at CST

- Station Area Improvement (SATIS) at Thane Station

The EMPs for these sub-projects were prepared in 1988 by AIC Watson Consultants Ltd.  These EMPs are currently being updated to encompass the changes in baseline data and site conditions that may have changed since the year 1998.  Thus, the EMPs presented here may undergo some modifications identified during the updating process.   The schedule for updating of EMPs for the above mentioned sub-projects are given in Table 7.1.

As mentioned in Table 7.1, EMPs for the remaining sub-projects will be prepared based on the related earlier EMPs.  The table also gives the schedule for completion. For instance, there are 30 pedestrian subway projects and 6 SATIS projects.   The EMPs presented for the CST pedestrian subway and Thane Station SATIS will be used to prepare EMPs for remaining projects in these categories.

EMP’s are developed for the above projects for the design, construction and operation phases of the projects and are given in Table 7.5 to 7.9 respectively attached behind this Chapter. The EMPs give the mitigation measures for the environmental impacts of the above projects summarised in Tables 7.4 (a) to (e).   The EMPs include references to the contract documents and responsibilities for implementation.  The EMPs are mainly prepared for the following significant environmental impacts/issues:

- Ambient Air Quality Deterioration

- Increase in Noise Levels

- Adverse Impacts on Ecology (Flora/Fauna)

- Population displacement

- Land degradation

- Surface and groundwater degradation

- Occupational health and safety

- Impacts on Traffic and road safety

In order to allow the local communities and road/rail users the benefit of a better corridor of movement and enhanced environmental setting, various site-specific environmental enhancement plans are prepared and are a part of the overall project design.   These enhancement measures typically include landscaping, roadside plantation, public seating at various locations, environmentally compatible  architecture etc.  These measures will be specified in the updated EA and EMP reports.

    - Environmental Monitoring Plan

Environmental monitoring plans have been specifically developed for MUTP sub-projects, involving monitoring sites, monitoring parameters, time and frequency, collection, analysis and the report of monitoring data.   The objectives of the monitoring plan are:

- To record the impact of MUTP on urban environmental quality during the construction and operation phases;

- To evaluate the effectiveness of the mitigation measures during the construction and operation phases.

- To satisfy the legal and community obligations

- To respond to the unanticipated environmental issues at an early stage and to verify the accuracy of environmental impact prediction.

At the project level, the vital parameters or performance indicators that will be monitored during construction and/or operation stages of the project include:

- Ambient air quality – SO2, PM10, CO, N0x

- Noise levels near sensitive locations

- Re-plantation success/ survival rates

- Traffic volume and characteristics

The recommended environmental quality-monitoring programme for the projects are given in Tables 7.10 to 7.14 attached behind this Chapter.    Methodology used for sampling and analysis will be as per prevalent requirements of Ministry of Environment and Forest and Indian Standard (IS) codes.  These monitoring plans are for the EMPs prepared in 1998 and are being updated along with the respective EMPs.

7.4	INSTITUTIONAL SETUP, COORDINATION & STRENGTHENING

7.4.1	Institutional Set-up and Coordination

The monitoring and evaluation of the EMPs are critical activities in implementation of the various projects.  Monitoring involves periodic checking to ascertain whether activities are going according to the plans.  It provides necessary feedback for project management team to keep the program on schedule.

The monitoring and reporting process for mitigation measures during construction will initiate from the contractor at the lowest rung who will report to the Project Implementation Agency (PIA) through the Project Management consultant.  The monitoring process and the roles and responsibilities of the various agencies for coordinating and managing the EMP implementation is described below :

Project Implementation Agencies (PIA) and Project Contractors

The responsibility of implementing the various environmental mitigation measures lies with the PIAs, such as Municipal Corporation of  Greater Mumbai, Public Works Department, Maharashtra State Road Development Corporation. The responsibility also includes various tasks such as notifying various affected parties such as the residents and commercial establishments, facilitate the relocation of people, notify other utility departments such as telephone, water supply, sewerage etc.  which use the road for providing public utility services.

The PIA will get the EMP implemented through the Project Management Consultant (PMC) by incorporating the EMP requirements in the contractual agreement along with the provision of penalties to be levied if the contractor fails to comply with the conditions. The contractor shall submit a report on compliance with the environmental mitigation measures (Environmental Compliance Reports, ECRs) before start of construction activities and periodically thereafter to the PIA.  The PIAs will then submit the ECRs to the JPD (Env), which after review and monitoring will be submitted to IMP through the Project Director, MUTP for confirmation of the implementation of the environmental mitigation measures.   The Project Director will accordingly submit the report to the World Bank. (Refer Figure 7.2 for the co-ordination model).

MMRDA

MMRDA as an apex organisation shall initiate co-ordination process among the concerned organisations for EMP implementation.  MMRDA shall take lead in:

- reviewing the progress of the projects and plans for the subsequent year - Institution wise

- reviewing and discussing the salient features of the reports in the year on environmental aspects/statistics like emission check and violations.

- organising and co-ordinating training programmes for all member organisations.

An Independent Monitoring Panel (IMP) has been constituted by MMRDA with the objective to ensure that the Bank’s policies related to social and environmental issues are followed.  The Chairman of IMP is Ex-Chief Secretary to Government of Maharashtra.  The other members are an eminent  Environmental Engineer, a senior Journalist and a leading Advocate.  The IMP will meet periodically to review the periodical reports, environmental compliance report, etc. submitted by PIAs and PMCs/Contractors.

MMRDA has proposed a co-ordination model (Refer Fig. 7.2) for promoting effective implementation of EMPs at sectoral level and at project level during construction and operation phases.  The responsibilities of key functionaries for EMP implementation are:

- The Project Director, MUTP has the overall responsibility of implementation of EMPs  and  co-ordination of all the environmental related matters of the transportation projects.

- The Joint Project Director (Environment) is responsible for both environmental planning and management. He will also be responsible for co-ordinating the environmental related works and ensuring preparation and implementation of sectoral and project level EMPs for the transportation projects.  In respect of R&R, he is particularly responsible for ensuring preparation and implementation of Community Environmental Management Plans (CEMP).

7.4.2	Reporting Requirements

Section 7.4.1 above describes institutional framework for EMP monitoring.  It is also important to identify the key parameters that have to be monitored and establish a reporting system.  The rationale for a reporting system is to ensure accountability for implementing the EMPs.

A generic reporting system proposed to be followed is given in Table 7.15.  However, the monitoring parameters and frequency of reporting are project specific and depend upon several factors such as:

- Environmental components affected

- Construction schedule of the project

- Environmental Management Plans suggested

Project specific monitoring Schedules as per format are given in Table 7.15 will be prepared and made available in the updated PLEA and MLEA reports for the projects.  Specific reporting formats will also be included.  The reporting sheets and formats will be made available in the project tender documents.

Figure 7.2    INSTITUTIONAL MECHANISM FOR IMPLEMENTATION OF EMP

High Power Steering Committee

IMP

Project Coordination

Committee

World Bank

MMRDA

Project Director MUTP

JPD (Environment)

MRVC

BEST

MCGM

PMC

PMC

PMC

PMC

Contractor

MSRDC/PWD

Contractor

Contractor

Contractor

    - Institutional Arrangements and Its Strengthening

The implementation of an environmentally sound transport strategy involves a number of institutions / organisations at various levels, with each organisation having a distinct role to play.  Introducing environmental dimensions in formulating and implementing a transportation strategy would require that these institutions shoulder additional responsibilities for ensuring that the strategy does not result in any significant adverse environmental impacts.

In order to examine the existing capacities and identify the additional responsibilities that the concerned Organisations/ institutions shall take up to address environmental issues, these Organisations are categorised into four groups – Apex Organisations, Project Implementation Agencies, Transport Service Organisations and Regulatory Organisations.

MMRDA is the apex organisation and being the regional planning authority has regular interactions with various Project Implementing Agencies. The existing capacities of these organisations for environmental management have been carefully assessed by MMRDA.  The envisaged roles and responsibilities of these organisations and additional strengthening requirements to meet the environmental obligations defined in this EA report are given in Table 7.16 attached at the end of this Chapter.

# 7.5      Cost Estimates for Mitigative Measures and Monitoring

The EMP implementation will require two types of financial commitments.  Budget required for strengthening the capacity of the concerned organisations responsible for implementing the plan; and moneys that will have to be spent for implementing mitigation measures identified for the individual road/rail projects under the transportation strategy.    These cost estimates are described separately below:

7.5.1	Cost Estimates for Institutional Capacity Building

The budget estimates provided in this section mainly focus on the capacity strengthening measures in terms of training. Training needs for building the capacity of the concerned organisations for environmental planning and management have been identified.  Table 7.17 lists these training programmes and also provides cost estimates for these programmes.  The total cost for the training programmes is estimated to be Rs. 4,80,000/-.

Table 7.17  Cost Estimated for Imparting Training (2001)

| Training Modules | Target Organisation | ### No of Batches | ## Cost/ Batch | ### Total cost Rs. |
| --- | --- | --- | --- | --- |
| Introduction to EIA (Duration – 3days) | MMRDA., MSRDC, MCGM, MRVC | 2 | 50,000 | 1,00,000 |
| Environmental Law (Duration – 3 days) | MMRDA., MSRDC, MCGM, MRVC | 4 | 30,000 | 1,20,000 |
| Air pollution and Noise Monitoring (Duration – 3days for introductory Course) | MMRDA., MSRDC, MCGM, MRVC | 2 | 30,000 | 60,000 |
| Duration 20 days for intensive course | MMRDA., MSRDC, MCGM, MRVC | 1 | 1,50,000 | 1, 50,000 |
| Environmental Planning and Decision Making – I (Duration – 3 days) | MMRDA, MCGM | 2 | 25,000 | 50,000 |
| Total Rs. 4,80,000/- |  |  |  |  |

Note :  One Batch may cover 15 to 20 participants. Cost per day of training is assumed at Rs.6000/-

7.5.2	Cost Estimates for Project Specific EMP Implementation

Provisions for these are expected to be made by the project proponent under the individual project budget.  Estimates for EMP implementation for the JVLR road project and BVQR rail project had been prepared in 1998 in the individual MLEA studies.   These estimates (including price escalations till  the year 2001) are given in Table 7.18.  The estimates have been prepared for EMPs related to the following tasks:

- Relocation of Utilities

- Compensatory and Road/Track Side Plantation

- Pollution control (dust, water pollution) during construction

- Noise level reduction

- Labour Camp sanitation

- Road safety

- Environmental Monitoring

The estimates for EMP implementation for the remaining projects such as   ROBs, SATIS and Pedestrian subways have been prepared based on the estimates of the  above 2 projects and engineering judgements.  A

consolidated estimate for EMP implementation for all related sub-projects are presented in Table 7.19.   The estimated cost of EMP implementation for MUTP sub-projects is Rs. 19.38 Million.

The above estimates are provisional and the final estimates will be available after the EMPs are updated (EMP updating is in progress).  The revised cost estimates for the projects will be presented in detail along with the updated EMP reports.  The final list of EMPs and respective cost estimates will be made a part of the bidding and contract documents.

7.6	CONTRACTUAL AGREEMENT

In order to facilitate smooth implementation of EMPs, the PIA shall include specific obligations in respect of EMP implementation in all tender and contract documents. The bidding and contract documents shall include tables (as updated for specific projects) given below at 1,2 and 3

- EMP requirements and obligations for the projects given in Tables 7.5 to 7.9.

- Environmental Monitoring Requirements given in Tables 7.10 to 7.14

- Reporting requirements given in Table 7.15

- Indicative EMP cost estimates as given in Table 7.18 and 7.19.

Table 7.18 Cost Estimates for EMP Implementation for JVLR and BVQR Sub-Projects

| # Component | EMP COST ESTIMATES |  |
| --- | --- | --- |
|  | JVLR | BVQR |
| Utility Relocation | 119600 (92000* + 30% Escalation) | 437000 (437000* + 30% Escalation) |
| Compensatory and Plantation | -- | 663000 510000* + 30 % Escalation |
| Road/Track Side Plantation | 2028000 1560000* + 20% Escalation | 280800 216000* + 30 % Escalation |
| Dust Suppression During Construction | 300000 30000/km x 10 km. | 390000 15000/km x 26 km. |
| Water Pollution – Silt Fencing Works | 10000 10000 per loc. x 1 Loc. | 150000 10000 per loc. x 15 Loc. |
| Noise barriers/raising boundary walls for Noise control | 1000000 5000/m x 200m | 1000000 5000/m x 200m |
| Labour Camp Sanitation | 100000 (10000/toilet x 10 toilets for 200 labourers) | 100000 (10000/toilet x 10 toilets for 200 labourers) |
| Road Safety (fencing etc.) | 50000 2500/meter x 20 m | 50000 2500/meter x 20 m |
| Safety and Noise awareness signage | 20000 (Lump sum) | 20000 (Lump sum) |
| Environmental Monitoring (Construction and Operating Stages) | 346000 | 145000 |
| ### Total | 3973600 | 3235800 |

*   Cost estimates of 1998

** Costs of R&R are separately covered in the RAP.

Table 7.19  Overall Cost Estimates for MUTP Sub-Projects

| Sr.No. | Name of the Project | EMP Cost Estimates |
| --- | --- | --- |
| Roads and traffic component |  |  |  |
| 1 | Jogeshwari-Vikhroli Link Road | 3973600 |
| 2 | Santacruz-Chembur link road | 3973600 |
| 3 | ROB projects at Vikhroli, Jogeshwari (South) and Jogeshwari (North) | 2477760 |
| 8 | 30 Pedestrian subways and bridges | 3000000 |
| 9 | 6 Station area traffic improvement schemes (SATIS) | 300000 |
| Rail Projects |  |  |
| 1 | Borivali Virar Quadrupling of Rails | 3235800 |
| 2 | 5th line between Santacruz and Borivali | 500000 |
| 3 | 5th and 6th lines between Kurla and Thane | 500000 |
| 6 | DC/AC conversion | 20000 |
| 13 | Technical Assistance, studies training | 480000 (from Table 7.18) |
| Sub Total |  | 18460760 |
| 5% Contingencies |  | 923038 |
| Grand Total |  | 19383798 |

Table 7.4 (a) Summary of Impacts for Jogeshwari-Vikhroli Link Road Project (JVLR)

| Environmental Component | Construction Stage Impacts | # Operation Stage Impacts |
| --- | --- | --- |
| Air Quality | Temporary increase in ambient SPM and N0x levels due to earthwork, handling construction material and use of fire wood for cooking by on-site laborers. | - No significant change in CO levels over the baseline. Predicted levels for 2011, well below the CO NAAQS. - N0x levels were predicted to be within NAAQS for 2011, except at some locations. - Both the baseline and predicted (for the year 2011) PM10 levels exceed the standards at most locations. |
| Noise Levels | Nuisance to nearby residents due to noise from construction activities such as operation of compressors, construction vehicles, concrete plants etc. | Day and night time predicted noise levels for the year 2011 range between 70.8 – 80.9 dB(A) and 58.6 – 73 dB(A) respectively. The day and night time noise standard will exceed upto 200 m from the road. |
| Water Quality and Hydrology | Temporary contamination of Mithi river due to surface runoff from widening of bridges over the river. Runoff could consist of contaminants such as soil, sanitary wastes and oil. | Mithi river contamination possible only due to accidental spillage of hazardous material from road tankers. No significant operation stage impact. |

| Ecological Issues | - Cutting/shifting of trees within 20 m of the road in the western section of JVLR. - None of the floral species are rare or endangered. | No significant adverse impact. |
| --- | --- | --- |
| Social (Rehabilitation and Resettlement) | In all 890 residential and commercial structures will be affected by the project. These will need to be relocated or compensated. | Nil |
| Land Degradation | High potential for soil erosion, change in topography, especially at the quarry sites. | Nil |

Table 7.4 (b) Summary of Impacts for Borivali – Virar Quadrupling of Rail Lines (BVQR) Project

| Environmental Component | Construction Stage Impacts | # Operation Stage Impacts |
| --- | --- | --- |
| Air Quality | Temporary increase in ambient SPM and N0x levels due to earthwork, handling construction material and use of fire wood for cooking by on-site laborers. | No adverse impact as electric trains will be operated. |
| Noise Levels | Nuisance to nearby residents due to noise from construction activities such as operation of compressors, construction vehicles, concrete plants etc. | Baseline noise levels already exceed noise standards. For the year 2011 the maximum increase in noise levels due to railway over the baseline noise at sensitive receptors is only 1.7 dB(A). The sensitive receptors are 2 hospitals and 2 educational institutes. Thus the impact is insignificant. |
| Water Quality and Hydrology | 15 major and minor bridges at various creek locations are planned. No obstruction of existing water courses is expected. Temporary contamination of creek water due to runoff from construction site, consisting of contaminants such as soil, sanitary wastes and oily wastes | No adverse impact expected. |

| Ecological Issues | - Trees within 20 m from the outermost track will be lost. - In addition mangrove vegetation will be lost near Jaffery creek, and Bassein creek. About 8500 numbers of Avicennia marina will be lost. | No significant adverse impact. |
| --- | --- | --- |
| Social (Rehabilitation and Resettlement) | In all 250 residential and commercial structures will be affected by the project. These will need to be relocated or compensated. | Nil |
| Land Degradation | High potential for soil erosion, change in topography especially at the quarry sites from where sand and stone will be sourced. | Nil |

Table 7.4 (c) Summary of Impacts for ROB AT VIKHROLI

| Environmental Component | Construction Stage Impacts | # Operation Stage Impacts |
| --- | --- | --- |
| Air Quality | Temporary increase in ambient SPM and N0x /CO levels due to earthwork, handling construction material and use of fire wood for cooking by on-site labourers. | - Predicted CO and N0x levels within NAAQS except for locations 10 m from the ROB. Baseline CO and N0x levels well within the standards. - Both the baseline and predicted (for the year 2011) PM10 levels exceed the standards. |
| Noise Levels | Nuisance to nearby residents and sensitive receptors (2 schools and a church) due to noise from construction activities such as operation of compressors, construction vehicles, concrete plants etc. | Day and night time predicted noise levels for the year 2011 ranged between 61.4 – 67.5 dB(A) and 57.3 – 63.4 dB(A) respectively. Overall noise levels are expected to increase from that of the base year. |

| Ecological Issues | - Clearance of minor road side vegetation - About 10 grown trees will be cut. - none of the floral species are of rare or endangered | No significant adverse impact. |
| --- | --- | --- |
| Social (Rehabilitation and Resettlement) | Dislocation of about 185 (G+1) structures with small business units in the ground floor and residential areas in upper floor. These will need to be relocated or compensated. | Nil |
| Land degradation | Temporary contamination of soil due to wastes from construction activities such as debris/oily wastes and sanitary wastes from labour camps, if any. | Nil |

Table 7.4 (d) Summary of Impacts for PEDESTRIAN SUBWAY AT CST.

| Environmental Component | Construction Stage Impacts | # Operation Stage Impacts |
| --- | --- | --- |
| Air Quality | Temporary increase in ambient SPM levels due to earthwork, handling construction material. | - Ambient Air quality in the area is expected to improve, due to significant easing of traffic congestion. - Both the baseline and predicted (for the year 2011) PM10 levels exceed the standards. |
| Noise Levels | Nuisance to nearby residents due to noise from construction activities such as operation of compressors, drilling work etc. | Average day and night time predicted noise levels for the year 2011 are 84.51 dB(A) and 78.49 dB(A) respectively. Noise levels are not expected to change significantly from baseline values. |

| Land degradation | Temporary contamination of soil due to wastes from construction activities such as debris/oily wastes and sanitary wastes from labour camps. | Nil |
| --- | --- | --- |
| Traffic and Transport | Disruption of existing traffic and pedestrian inconvenience at CST junction. | Substantial benefits from improved vehicle speeds, reduction in traffic congestion, faster pedestrian movement. |

Table 7.4 (e) Summary of Impacts for Station Area Improvement Scheme (SATIS) at Thane West

| Environmental Component | Construction Stage Impacts | # Operation Stage Impacts |
| --- | --- | --- |
| Air Quality | Insignificant – Minimal construction activity. | Baseline CO, PM10 and HC levels exceed NAAQ standards. No major beneficial impact on AAQ. Existing AAQ may further worsen due to increase in vehicular traffic. |
| Noise Levels | Marginal increase in noise levels due to noise from construction activities. | Average day and night time predicted noise levels for the year 2001 are 78.5 dB(A) and 72.5 dB(A) respectively. Noise levels are not expected to change significantly from baseline values. |

| Traffic and Transport | Disruption of existing traffic and pedestrian inconvenience at CST junction. | Improved vehicle speeds, reduction in traffic congestion, faster pedestrian movement. |
| --- | --- | --- |

TABLE 7.5 Environmental Management Plan for Jogeshwari-Vikhroli Link Road Project (JVLR)

# PRE-CONSTRUCTION / DESIGN STAGE

| Environmental Impact | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Ecological impacts such as tree cutting and destruction of mangroves | Trees to be removed before commencement of construction, with prior permission from MCGM. Road alignment shall be selected in such a way that there is minimum destruction of trees. | Legal requirement. | Before Start of construction | Contractor / PMC | PIA |
| Resettlement and rehabilitation | The acquisition of land and property shall be carried out in accordance to the RAP and entitlement framework of the project. It shall be ensured that all R&R activities be reasonably completed as per RAP, before the construction activity starts in the relevant section. | RAP Requirement | Before Start of construction of relevant section. | MMRDA | MMRDA |
| Utility relocation | All utilities, such as electrical installations, telephone lines etc. to be shifted after prior approval of agencies. | Project requirement | Before Start of construction | PIA | MMRDA |
| Noise pollution | Based on noise monitoring results, noise control plans to be prepared. The plans shall include the installation of noise barriers including additional road side plantation where practical. | Project requirement | Before Start of construction | PIA | MMRDA |

# TABLE 7.5 Cont..

# CONSTRUCTION STAGE EMP (JVLR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Generation of Dust due to construction activity | All vehicles delivering material to the site shall be covered to avoid material spillage. While unloading material, fall height shall be kept low to minimize fugitive dust generation. | MOST:111.9 | During construction | Contractor / PMC | PIA, MMRDA |
|  | Contractor shall take precaution to reduce dust emissions from the hot mix plants. Emissions should meet emission control legislation. | MOST 111.5 |  |  |  |
|  | Construction site to be watered periodically to minimize fugitive dust generation. |  |  |  |  |
|  | All existing highways and roads used by the vehicles of contractor and his team shall be kept clean and clear of all dust, mud and other extraneous material dropped by such vehicles. | MOST:111.9 |  |  |  |
|  | All Earthwork and construction material should be protected in such a manner so as to minimize generation of dust. | Contract Documents |  |  |  |
| Construction Vehicle/equipm-ent exhaust | Exhaust emissions from all construction vehicles shall adhere to vehicle emission norms. | Contract requirement | During construction | Contractor / PMC | PIA, MMRDA |
| Noise from vehicles, plants and equipment | All vehicle and construction equipment shall be fitted with exhaust silencers. Damaged silencers to be promptly replaced by contractor. | MOST 111.1 | During construction |  |  |
|  | All construction equipment (particularly DG sets, shall adhere to noise standards of Ministry of Environment and Forests. | MOST:111.13 |  |  |  |
|  | Contractor shall ensure that noise levels near residential areas are within the day time and night time noise standards under the Environmental (Protection) Rules, 1986 and its amendments. | MOST:111.13 |  |  |  |
|  | Operation of noise generating equipment such as pile driving, concrete and drilling etc. shall be limited to day time hours. | Contract requirement |  |  |  |

# TABLE 7.5 Cont..

# CONSTRUCTION STAGE EMP (JVLR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Noise from vehicles, plants and equipment | Workers exposed to loud noise (As per Factory Act requirements) shall wear earplugs/earmuffs | MOST: 111.6 | During construction | Contractor / PMC | PIA, MMRDA |
| Contamination of Mithi River and other water bodies. | Silt fencing shall be provided as the base of the embankment for the entire perimeter of any water body (including wells) | MOST : 306 | During construction | Contractor /PMC | PIA, MMRDA |
|  | Construction material containing fine particles shall be stored in an enclosure such that sediment laden water does not drain into nearby water courses. | MOST: 306 |  |  |  |
|  | Siltation of soil into water bodies shall be prevented as far as possible by adapting soil erosion control measures as per MoST guidelines | MOST: 305.2.2.2 MOST: 306.2 |  |  |  |
|  | Construction work close to streams or water bodies shall be avoided during monsoon. | MOST 111.13 |  |  |  |
|  | In sections along water courses and close to cross drainage channels, earth, stone or any other construction material shall be properly disposed so as not to block the flow of water. |  |  |  |  |
| Ecological impacts such as tree cutting and destruction of mangroves | All works shall be carried out in such a fashion that damage to flora is minimum | Contract requirement | During construction | Contractor/ PMC | PIA, MMRDA |
|  | Trees and other flora shall be cut only after receiving clearances from appropriate agency |  |  |  |  |
|  | Road side plantation shall be done as per pre-approved plan. |  |  |  |  |
| Soil erosion | On road embankments, slopes shall be stabilized. The work shall consist of measures as per design, or as directed by the engineer to control soil erosion, sedimentation and water pollution, through use of berms, dikes, sediment basin, mulches, grasses, slope drains and other devices. | MOST : 306.2 / Design document | During construction | Contractor / PMC | PIA, MMRDA |

# TABLE 7.5 Cont..

# CONSTRUCTION STAGE EMP (JVLR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Debris disposal | Debris generated due to dismantling of existing pavement/structures shall be suitably reused in proposed construction. Unutilisable debris shall be suitably disposed either as fill material or at pre-designated dump locations, approved by engineer. Care should be taken that the material does not block natural drainage or contaminate water bodies. | MOST: 112.6 MOST :112.2 | During construction | Contractor / PMC | PIA, MMRDA |
|  | Bentonite slurry or similar debris generated from pile driving or other construction activities shall be disposed such that it does not flow into surface water bodies or form mud puddles in the area. |  |  |  |  |
| Loss of top soil | The top soil from all areas of cutting and all areas to be permanently covered shall be stripped to a specified depth of 150 mm and stored in stock piles. The top soil from the stock pile shall be used to cover disturbed areas and cut slopes and also for re-development of borrow areas, landscaping and road side plantation. | MOST : 300 | During construction | Contractor/ PMC | PIA, MMRDA |
| Soil contamination by construction wastes, fuel etc. | Oil and fuel spills from construction equipment shall be minimized by good O&M practices. Soils contaminated by such spills shall be disposed as per MOEF requirements. | Contract Document | During construction | Contractor / PMC | PIA, MMRDA |
| Land degradation in quarry | Sand and stones shall be sourced from only pre-approved and licensed quarries. It should be assured that these quarries have a quarry rehabilitation plan. | MOST 111.3 | During construction | Contractor/ PMC | PIA, MMRDA |

# TABLE 7.5 Cont..

# CONSTRUCTION STAGE EMP (JVLR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Health and hygiene at construction labour camps | Construction labour camps shall be located at least 200 m away from the nearest habitation to avoid conflicts and stress over infrastructure facilities. | Contract Document | During construction | Contractor / PMC | PIA, MMRDA |
|  | The contractor will provide, erect and maintain necessary (temporary) living accommodation and ancillary facilities such as toilet blocks, potable water supply etc. for labour as per the standards set by the Building and other construction Workers (Regulation of Employment and Conditions of Service) Act, 1996 and shall be approved by the engineer. |  |  |  |  |
|  | On completion of the works, all such temporary structures shall be cleared away along with all wastes. All excreta and other disposal pits should be filled in and effectively sealed. The site should be left clean and tidy to the satisfaction of the engineer. |  |  |  |  |
| Traffic delays and congestion | Detailed traffic control plan shall be prepared 5 days prior to commencement of works on any section of the road. The control plans shall contain details of temporary diversions, details of arrangement of construction under traffic. | MOST: 112.1 | Before Construction | PIA | MMRDA |
|  | Special consideration shall be given in preparation of traffic control plan to the safety of pedestrians and workers at night. | MOST: 112.2 |  | PIA | MMRDA |
|  | Temporary traffic detours shall be kept free of dust by frequent application of water, if necessary. | MOST: 112.5 |  | Contractor / PMC | PIA |
| Traffic Control and Safety | The contractor shall take all necessary measures for the safety of traffic during construction and provide, erect and maintain such barricades, including signs, markings, flags, lights and flagmen as may be required by the engineer for the information and protection of traffic. All such measures should be as per requirement of MOST. | MOST: 112.4 MOST: 112.1 | Before Construction | Contractor / PMC | PIA |

# TABLE 7.5 Cont..

# CONSTRUCTION STAGE EMP (JVLR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Occupational Health and Safety | The contractor is required to comply with all the precautions as required for the safety of workmen as per the International Labour Organisation (ILO) Convention No. 62, as far as those are applicable to the contract. | Contract Requirement | During Construction | Contractor / PMC | PIA, MMRDA |
|  | The contractor shall supply all necessary safety appliances such as safety goggles, helmets, safety belts, ear plugs, masks etc. to the worker and staff. All laws related to safe scaffolding, ladders, working platform, gangway, stairwells, excavations, safety entry and exit etc. shall be complied with. |  |  |  |  |
|  | Adequate precautions shall be taken to prevent danger from electrical equipment. All machines/equipment used shall confirm to the relevant Indian Standards (IS) codes and shall be regularly inspected by the engineer. | MOST: 106 | During Construction | Contractor / PMC | PIA, MMRDA |
|  | A readily available first aid unit including an adequate supply of sterilized dressing material and appliances shall be provided as per the requirements under the Factory Act. | Contract Requirement | During Construction | Contractor / PMC | PIA, MMRDA |
|  | All anti-malarial measures as prescribed by the engineer shall be complied with, including filling up of burrow pits. | Contract Requirement | During Construction | Contractor/ PMC | PIA, MMRDA |

# TABLE 7.5 Cont..

# OPERATION STAGE EMP (JVLR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Atmospheric pollution | Ambient air concentrations of various pollutants shall be monitored as per the pollution monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency (MCGM) | MMRDA , World Bank |
|  | Vehicle exhaust standards shall be enforced strictly. |  |  |  |  |
|  | Road side plantation to be maintained |  |  |  |  |
| Noise pollution | Monitoring of noise levels at sensitive receptors as per monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency. (MCGM) RTO, PIA | MMRDA , World Bank |
|  | Public awareness campaigns for noise reduction including placing adequate “No Honking” sign boards at sensitive locations. |  |  |  |  |
| Land and water contamination from accidental spills | Contingency plans to be in place for spill clean-up | Project requirement | Starting immediately after completion of construction | RTO, PIA | MMRDA |
|  | Spill prevention and waste fuel/oil treatment and disposal to be made mandatory for utilities such as auto-service station, petrol pumps along the entire stretch of the construction. |  |  |  |  |
| Storm water and drain maintenance | All drains to be maintained and cleaned periodically. | Project requirement | Starting immediately after completion of construction | Municipal authority, PIA | MMRDA |
| Traffic and safety | Traffic management plans shall be prepared. | Project requirement | Through operation stage | PIA, Development Authorities | MMRDA |
|  | Road control width to be enforced. Unauthorised building development to be checked. Slum encroachments to be discouraged. |  |  |  |  |
|  | Traffic control measures including speed limits to be enforced strictly. |  |  |  |  |
|  | Traffic volume and speed to be monitored to record benefits achieved from the project. |  |  |  |  |

TABLE 7.6 Environmental Management Plan for Borivali – Virar Quadrupling of Rail Lines (BVQR) Project

# PRE-CONSTRUCTION / DESIGN STAGE

| Environmental Impact | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Ecological impacts such as tree cutting and destruction of mangroves. | Trees to be removed before commencement of construction, with prior permission from MCGM / Forest Department.. | Legal requirement. | Before Start of construction | Contractor/PMC | PIA |
| Resettlement and rehabilitation | The acquisition of land and property shall be carried out in accordance to the RAP and entitlement framework of the project. It shall be ensured that all R&R activities be reasonably completed as per RAP, before the construction activity starts | RAP Requirement | Before Start of construction | MMRDA/PIA | MMRDA |
| Utility relocation | All utilities, such as electrical installations, telephone lines etc. to be shifted after prior approval of agencies. | Project requirement | Before Start of construction | PIA | MMRDA |
| Noise pollution | Noise control plans to be prepared. This shall include installation of noise barriers at sensitive receptors. | Project requirement | Before Start of construction | PIA | MMRDA |

# TABLE 7.6 Cont..

# CONSTRUCTION STAGE EMP (BVQR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Generation of Dust due to construction activity | All vehicles delivering material to the site shall be covered to avoid material spillage. While unloading material, fall height shall be kept low to minimize fugitive dust generation. | MOST:111.9 | During construction During construction | Contractor /PMC | PIA, MMRDA |
|  | Construction site to be watered periodically to minimize fugitive dust generation. |  |  |  |  |
|  | All existing highways and roads used by the vehicles of contractor and his team shall be kept clean and clear of all dust, mud and other extraneous material dropped by such vehicles. | MOST:111.9 |  |  |  |
|  | All Earthwork and construction material should protected in such a manner so as to minimize generation of dust. | Contract Documents |  |  |  |
| Construction Vehicle/equipm-ent exhaust | Exhaust emissions from all construction vehicles shall adhere to vehicle emission norms. | Contract requirement | During construction | Contractor /PMC Contractor /PMC | PIA, MMRDA PIA, MMRDA |
| Noise from vehicles, plants and equipment | All vehicle and construction equipment shall be fitted with exhaust silencers. Damaged silencers to be promptly replaced by contractor. | MOST 111.1 | During construction |  |  |
|  | All construction equipment (particularly DG sets, shall adhere to noise standards of Ministry of Environment and Forests. | MOST:111.13 |  |  |  |
|  | Contractor shall ensure that noise levels near residential areas are within the day time and night time noise standards under the Environmental (Protection) Rules, 1986 and its amendments. | MOST:111.13 |  |  |  |
|  | Operation of noise generating equipment such as compacting, drilling, blasting etc. shall be limited to day time hours. | Contract requirement |  |  |  |

## TABLE 7.6 Cont.

CONSTRUCTION STAGE EMP (BVQR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Noise from vehicles, plants and equipment | Workers exposed to loud noise (As per Factory Act requirements) shall wear earplugs/earmuffs | MOST: 111.6 | During construction | Contractor /PMC | PIA, MMRDA |
| Contamination of creek water at various bridge locations and other water bodies such as wells and lakes/ponds. | Silt fencing shall be provided at the base of the embankment for the entire perimeter of any water body (including wells) | MOST : 306 | During construction | Contractor /PMC | PIA, MMRDA |
|  | Construction material containing fine particles shall be stored in an enclosure such that sediment laden water does not drain into nearby water courses. | MOST: 306 |  |  |  |
|  | Siltation of soil into water bodies shall be prevented as far as possible by adapting soil erosion control measures as per MoST guidelines | MOST: 305.2.2.2 MOST: 306.2 |  |  |  |
|  | Construction work close to water bodies shall be avoided during monsoon. | MOST 111.13 |  |  |  |
|  | In sections along water courses and close to cross drainage channels, earth, stone or any other construction material shall be properly disposed so as not to block the flow of water. |  |  |  |  |
| Ecological impacts such as tree cutting and destruction of mangroves | All works shall be carried out in such a fashion that damage to flora is minimum | Contract requirement | During construction | Contractor/PMC | PIA, MMRDA |
|  | Trees and other flora shall be cut only after receiving clearances from appropriate agency |  |  |  |  |
|  | Track - side plantation shall be done as per pre-approved plan. |  |  |  |  |
| Soil erosion | On road/track embankments, slopes shall be stabilized. The work shall consist of measures as per design, or as directed by the engineer to control soil erosion, sedimentation and water pollution, through use of berms, dikes, sediment basin, mulches, grasses, slope drains and other devices. | MOST : 306.2 / Design document | During construction | Contractor/PMC | PIA, MMRDA |

## TABLE 7.6 Cont.

CONSTRUCTION STAGE EMP (BVQR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Debris disposal | Debris generated due to dismantling of existing pavement/structures shall be suitably reused in proposed construction. Unutilisable debris shall be suitably disposed either as fill material or at pre-designated dump locations, approved by engineer. Care should be taken that the material does not block natural drainage or contaminate water bodies. | MOST: 112.6 MOST :112.2 | During construction | Contractor/PMC | PIA, MMRDA |
|  | Bentonite slurry or similar debris generated from pile driving or other construction activities shall be disposed such that it does not flow into surface water bodies or form mud puddles in the area. |  |  |  |  |
| Loss of top soil | The top soil from all areas of cutting and all areas to be permanently covered shall be stripped to a specified depth of 150 mm and stored in stock piles. The top soil from the stock pile shall be used to cover disturbed areas and cut slopes and also for re-development of borrow areas, landscaping and road side plantation. | MOST : 300 | During construction | Contractor/PMC | PIA, MMRDA |
| Soil contamination by construction wastes, fuel etc. | Oil and fuel spills from construction equipment shall be minimized by good O&M practices. Soils contaminated by such spills shall be disposed as per MOEF requirements. | Contract Document | During construction | Contractor/PMC | PIA, MMRDA |
| Land degradation in quarry | Sand and stones shall be sourced from only pre-approved and licensed quarries. It should be assured that these quarries have a quarry rehabilitation plan. | MOST 111.3 | During construction | Contractor/PMC | PIA, MMRDA |

## TABLE 7.6 Cont.

CONSTRUCTION STAGE EMP (BVQR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Health and hygiene at construction labour camps | Construction labour camps shall be located at least 200 m away from the nearest habitation to avoid conflicts and stress over infrastructure facilities. | Contract Document | During construction | Contractor/PMC | PIA, MMRDA |
|  | The contractor will provide, erect and maintain necessary (temporary) living accommodation and ancillary facilities such as toilet blocks, potable water supply etc. for labour as per the standards set by the Building and other construction Workers (Regulation of Employment and Conditions of Service) Act, 1996 and shall be approved by the engineer. |  |  |  |  |
|  | On completion of the works, all such temporary structures shall be cleared away along with all wastes. All excreta and other disposal pits should be filled in and effectively sealed. The site should be left clean and tidy to the satisfaction of the engineer. |  |  |  |  |
| Delays and congestion on tracks | The construction plan shall be discussed in advance with Western Railway Authorities to avoid delays in existing train schedules. | MOST: 112.1 | Before Construction | Contractor/PMC | PIA |
|  | Special consideration shall be given in preparation of traffic control plan to the safety of pedestrians (at railway crossings) and workers at night. | MOST: 112.2 |  |  |  |
| Traffic Control and Safety | The contractor shall take all necessary measures for the safety of existing train traffic during construction and provide, erect and maintain such barricades, including signs, markings, flags, lights and flagmen as may be required by the engineer for the information and protection of traffic. All such measures should as per requirement of Western Railway. | Contract Document | During construction | Contractor/PMC | PIA |

## TABLE 7.6 Cont.

CONSTRUCTION STAGE EMP (BVQR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Occupational Health and Safety | The contractor is required to comply with all the precautions as required for the safety of workmen as per the International Labour Organisation (ILO) Convention No. 62, as far as those are applicable to the contract. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |
|  | The contractor shall supply all necessary safety appliances such as safety goggles, helmets, safety belts, ear plugs, masks etc. to the worker and staff. All laws related to safe scaffolding, ladders, working platform, gangway, stairwells, excavations, safety entry and exit etc. shall be complied with. |  |  |  |  |
|  | Adequate precautions shall be taken to prevent danger from electrical equipment. All machines/equipment used shall confirm to the relevant Indian Standards (IS) codes and shall be regularly inspected by the engineer. | MOST: 106 | During Construction | Contractor/PMC | PIA, MMRDA |
|  | A readily available first aid unit including an adequate supply of sterilized dressing material and appliances shall be provided as per the requirements under the Factory Act. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |
|  | All anti-malarial measures as prescribed by the engineer shall be complied with, including filling up of burrow pits. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |

## TABLE 7.6 Cont.

OPERATION STAGE EMP (BVQR PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- |
|  |  |  | Implementation | Supervision |
| Noise pollution | Monitoring of noise levels at sensitive receptors as per monitoring plan. | Starting immediately after completion of construction | Pollution monitoring agency - MCGB | MMRDA , World Bank |
|  | Maintenance of track-side plantation |  | Western Railway | MMRDA |
| Land and water contamination from accidental spills from goods trains. | Contingency plans to be in place for spill clean-up | Starting immediately after completion of construction | Western Railway | MMRDA |
| Rail traffic and safety | Unauthorised building development to be checked. Slum encroachments to be discouraged. | Through operation stage | PIA, Development Authorities, Western Railway | MMRDA |
| Loss of mangrove and track-side plantations | Regular maintenance of compensatory plantation done. | Through operation stage | Western Railway | MMRDA |

TABLE 7.7 Environmental Management Plan for ROB at Vikhroli

# PRE-CONSTRUCTION / DESIGN STAGE

| Environmental Impact | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Ecological impacts such as tree cutting | Trees to be removed before commencement of construction, with prior permission from MCGB / Forest Department.. Road alignment shall be selected in such a way that there is minimum destruction of trees. | Legal requirement. | Before Start of construction | Contractor/PMC | PIA |
| Resettlement and rehabilitation | The acquisition of land and property shall be carried out in accordance to the RAP and entitlement framework of the project. It shall be ensured that all R&R activities be reasonably completed as per RAP, before the construction activity starts | RAP Requirement | Before Start of construction | MMRDA | MMRDA |
| Utility relocation | All utilities, such as electrical installations, telephone lines etc. to be shifted after prior approval of agencies. | Project requirement | Before Start of construction | PIA | MMRDA |
| Noise pollution | Noise control plans to be prepared. This shall include the installation of noise barriers including additional road side plantation where practical (refer PLEA report of March 1998 for barrier design details) | Legal requirement. | Before Start of construction | PIA | MMRDA |

# TABLE 7.7 Cont..

# CONSTRUCTION STAGE EMP (VIKHROLI  ROB  PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Generation of Dust due to construction activity | All vehicles delivering material to the site shall be covered to avoid material spillage. While unloading material, fall height shall be kept low to minimize fugitive dust generation. | MOST:111.9 | During construction | Contractor/PMC | PIA, MMRDA |
|  | Contractor shall take precaution to reduce dust emissions from the hot mix plants. Emissions should meet emission control legislation. | MOST 111.5 |  |  |  |
|  | Construction site to be watered periodically to minimize fugitive dust generation. |  |  |  |  |
|  | All existing highways and roads used by the vehicles of contractor and his team shall be kept clean and clear of all dust, mud and other extraneous material dropped by such vehicles. | MOST:111.9 |  |  |  |
|  | All Earthwork and construction material should protected in such a manner so as to minimize generation of dust. | Contract Documents |  |  |  |
| Construction Vehicle/equipm-ent exhaust | Exhaust emissions from all construction vehicles shall adhere to vehicle emission norms. | Contract requirement | During construction | Contractor/PMC | PIA, MMRDA |
| Noise from vehicles, plants and equipment | All vehicle and construction equipment shall be fitted with exhaust silencers. Damaged silencers to be promptly replaced by contractor. | MOST 111.1 | During construction |  |  |
|  | All construction equipment (particularly DG sets, shall adhere to noise standards of Ministry of Environment and Forests. | MOST:111.13 |  |  |  |
|  | Contractor shall ensure that noise levels near residential areas are within the day time and night time noise standards under the Environmental (Protection) Rules, 1986 and its amendments. | MOST:111.13 |  |  |  |
|  | Operation of noise generating equipment such as pile driving, concrete and drilling etc. shall be limited to day time hours. | Contract requirement |  |  |  |

# TABLE 7.7 Cont..

# CONSTRUCTION STAGE EMP (VIKHROLI  ROB  PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Noise from vehicles, plants and equipment | Workers exposed to loud noise (As per Factory Act requirements) shall wear earplugs/earmuffs | MOST: 111.6 | During construction | Contractor/PMC | PIA, MMRDA |
|  | Noise barriers and other noise attenuation measures, as prescribed in the Noise Control Plan (design stage) to be constructed/installed. |  | During construction | Contractor/PMC | PIA, MMRDA |
| Contamination of Mithi River and other water bodies. | Silt fencing shall be provided as the base of the embankment for the entire perimeter of any water body (including wells) | MOST : 306 | During construction | Contractor/PMC | PIA, MMRDA |
|  | Construction material containing fine particles shall be stored in an enclosure such that sediment laden water does not drain into nearby water courses. | MOST: 306 |  |  |  |
|  | Siltation of soil into water bodies shall be prevented as far as possible by adapting soil erosion control measures as per MoST guidelines | MOST: 305.2.2.2 MOST: 306.2 |  |  |  |
|  | Construction work close to streams or water bodies shall be avoided during monsoon. | MOST 111.13 |  |  |  |
|  | In sections along water courses and close to cross drainage channels, earth, stone or any other construction material shall be properly disposed so as not to block the flow of water. |  |  |  |  |
| Ecological impacts such as tree cutting | All works shall be carried out in such a fashion that damage to flora is minimum | Contract requirement | During construction | Contractor/PMC | PIA, MMRDA |
|  | Trees and other flora shall be cut only after receiving clearances from appropriate agency |  |  |  |  |
|  | Road side plantation shall be done as per pre-approved plan. |  |  |  |  |

TABLE 7.7 Cont..

# CONSTRUCTION STAGE EMP (VIKHROLI  ROB  PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Debris disposal | Debris generated due to dismantling of existing pavement/structures shall be suitably reused in proposed construction. Unutilisable debris shall be suitably disposed either as fill material or at pre-designated dump locations, approved by engineer. Care should be taken that the material does not block natural drainage. | MOST: 112.6 MOST :112.2 | During construction | Contractor/PMC | PIA, MMRDA |
|  | Bentonite slurry or similar debris generated from pile driving or other construction activities shall be disposed such that it does not flow into surface water bodies (wells) or form mud puddles in the area. |  |  |  |  |
| Soil contamination by construction wastes, fuel etc. | Oil and fuel spills from construction equipment shall be minimized by good O&M practices. Soils contaminated by such spills shall be disposed as per MOEF requirements. | Contract Document | During construction | Contractor | PIA, MMRDA |
| Land degradation in quarry | Sand and stones shall be sourced from only pre-approved and licensed quarries. It should be assured that these quarries have a quarry rehabilitation plan. | MOST 111.3 | During construction | Contractor | PIA, MMRDA |
| Health and hygiene at construction labour camps | Construction labour camps shall be located at least 200 m away from the nearest habitation to avoid conflicts and stress over infrastructure facilities. | Contract Document | During construction | Contractor | PIA, MMRDA |
|  | The contractor will provide, erect and maintain necessary (temporary) living accommodation and ancillary facilities such as toilet blocks, potable water supply etc. for labour as per the standards set by the Building and other construction Workers (Regulation of Employment and Conditions of Service) Act, 1996 and shall be approved by the engineer. |  |  |  |  |

# TABLE 7.7 Cont..

# CONSTRUCTION STAGE EMP (VIKHROLI  ROB  PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
|  | On completion of the works, all such temporary structures shall be cleared away along with all wastes. All excreta and other disposal pits should be filled in and effectively sealed. The site should be left clean and tidy to the satisfaction of the engineer. |  |  |  |  |
| Traffic delays and congestion | Detailed traffic control plan shall be prepared 5 days prior to commencement of works on any section of the road. The control plans shall contain details of temporary diversions, details of arrangement of construction under traffic. | MOST: 112.1 | Before Construction | PIA | MMRDA |
|  | Special consideration shall be given in preparation of traffic control plan to the safety of pedestrians and workers at night. | MOST: 112.2 |  | PIA | MMRDA |
|  | Temporary traffic detours shall be kept free of dust by frequent application of water, if necessary. | MOST: 112.5 |  | Contractor/PMC | PIA |
| Traffic Control and Safety | The contractor shall take all necessary measures for the safety of traffic during construction and provide, erect and maintain such barricades, including signs, markings, flags, lights and flagmen as may be required by the engineer for the information and protection of traffic. All such measures should be as per requirement of MOST. | MOST: 112.4 MOST: 112.1 | During construction | Contractor/PMC | PIA |

TABLE 7.7 Cont..

# CONSTRUCTION STAGE EMP (VIKHROLI  ROB  PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Occupational Health and Safety | The contractor is required to comply with all the precautions as required for the safety of workmen as per the International Labour Organisation (ILO) Convention No. 62, as far as those are applicable to the contract. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |
|  | The contractor shall supply all necessary safety appliances such as safety goggles, helmets, safety belts, ear plugs, masks etc. to the worker and staff. All laws related to safe scaffolding, ladders, working platform, gangway, stairwells, excavations, safety entry and exit etc. shall be complied with. |  |  |  |  |
|  | Adequate precautions shall be taken to prevent danger from electrical equipment. All machines/equipment used shall confirm to the relevant Indian Standards (IS) codes and shall be regularly inspected by the engineer. | MOST: 106 | During Construction | Contractor/PMC | PIA, MMRDA |
|  | A readily available first aid unit including an adequate supply of sterilized dressing material and appliances shall be provided as per the requirements under the Factory Act. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |
|  | All anti-malarial measures as prescribed by the engineer shall be complied with, including filling up of burrow pits. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |

# TABLE 7.7 Cont..

# OPERATION STAGE EMP (VIKHROLI  ROB  PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Atmospheric pollution | Ambient air concentrations of various pollutants shall be monitored as per the pollution monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency - MCGB | MMRDA |
|  | Vehicle exhaust standards shall be enforced strictly. |  |  |  |  |
|  | Road side plantation to be maintained |  |  |  |  |
| Noise pollution | Monitoring of noise levels at sensitive receptors as per monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency. RTO, PIA RTO, PIA | MMRDA |
|  | Public awareness campaigns for noise reduction including placing adequate “No Honking” sign boards at sensitive locations. |  |  |  |  |
|  | Restriction of heavy vehicles during day time only. |  |  |  |  |
|  | Spill prevention and waste fuel/oil treatment and disposal to be made mandatory for utilities such as auto-service station, petrol pumps along the entire stretch of the construction. |  |  |  |  |
| Storm water and drain maintenance | All drains to be maintained and cleaned periodically. | Project requirement | Starting immediately after completion of construction | Municipal authority, PIA | MMRDA |
| Traffic and safety | Traffic management plans shall be prepared. | Project requirement | Through operation stage | PIA, Development Authorities | MMRDA |
|  | Traffic control measures including speed limits to be enforced strictly. |  |  |  |  |
|  | Traffic density to be monitored to record benefits achieved from the project. |  |  |  |  |

TABLE 7.8 Environmental Management Plan for CST PEDESTRIAN SUBWAY PROJECT

# PRE-CONSTRUCTION / DESIGN STAGE

| Environmental Impact | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Traffic delays and congestion | Detailed traffic control plan shall be prepared prior to commencement of works on any section of the road. The control plans shall contain details of temporary diversions, details of arrangement of construction under traffic. | MOST: 112.1 | Before Construction | PIA | MMRDA |
|  | Special consideration shall be given in preparation of traffic control plan to the safety of pedestrians. | MOST: 112.2 | Before Construction | PIA | MMRDA |
|  | Selection of construction technology that minimises in situ work that cause disruption to the traffic. | MOST: 112.1 | Before Construction | PIA | MMRDA |
| Utility protection and relocation. | Plans of all underground utilities, such as sewerage, water pipelines etc. shall be obtained and analysed prior to start of work. Related agencies shall be informed prior to start of excavation/construction. | Project requirement | Before Start of construction | PIA | MMRDA |

# TABLE 7.8 Cont..

# CONSTRUCTION STAGE EMP (CST PEDESTRIAN SUBWAY PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents |  | Time Frame |  | Responsibility |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Implementation |  |  | Supervision |
| Generation of Dust due to construction activity | All vehicles carrying excavated material shall be covered to avoid material spillage. While unloading material, fall height shall be kept low to minimize fugitive dust generation. | MOST:111.9 |  | During construction |  | Contractor/PMC |  |  | PIA, MMRDA |
|  | Water sprinkling to minimize dust from excavated material. | Contract Documents |  |  |  |  |  |  |  |
|  | Construction site to be suitably enclosed to contain fugitive dust emissions. |  |  |  |  |  |  |  |  |
|  | All Earthwork and construction material should protected in such a manner so as to minimize generation of dust. |  |  |  |  |  |  |  |  |
| Construction Vehicle/equipm-ent exhaust | Exhaust emissions from all construction equipment (DG sets, compressors etc.) shall adhere to emission norms. | Contract requirement |  | During construction |  | Contractor/PMC |  |  | PIA, MMRDA |
| Noise from vehicles, plants and equipment Noise from vehicles, plants and equipment | All vehicle and construction equipment shall be fitted with exhaust silencers. Damaged silencers to be promptly replaced by contractor. | MOST 111.1 |  | During construction |  |  |  |  |  |
|  | All construction equipment (particularly DG sets, shall adhere to noise standards of Ministry of Environment and Forests. | MOST:111.13 |  |  |  |  |  |  |  |
|  | Operation of noise generating equipment such as pile driving, concrete and drilling etc. shall be done during night time as this area is predominantly pubic and commercial where day time activity is more than night time. | Contract requirement |  |  |  |  |  |  |  |
|  | Workers exposed to loud noise (As per Factory Act requirements) shall wear earplugs/earmuffs |  | MOST: 111.6 |  | During construction |  | Contractor/PMC | PIA, MMRDA |  |

# TABLE 7.8 Cont..

# CONSTRUCTION STAGE EMP (CST PEDESTRIAN SUBWAY PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Debris disposal | Debris generated from excavated material and due to dismantling of existing pavement/structures shall be suitably reused in proposed construction. Unutilisable debris shall be suitably disposed as fill material at an approved location elsewhere. | MOST: 112.6 MOST :112.2 | During construction | Contractor/PMC | PIA, MMRDA |
| Traffic delays and congestion | Detailed traffic control plan shall be prepared prior to commencement of works on any section of the road. The control plans shall contain details of temporary diversions, details of arrangement of construction under traffic. | MOST: 112.1 | Before Construction | PIA | MMRDA |
|  | Special consideration shall be given in preparation of traffic control plan to the safety of pedestrians. | MOST: 112.2 |  | PIA | MMRDA |
| Traffic Control and Safety | The contractor shall take all necessary measures for the safety of traffic during construction and provide, erect and maintain such barricades, including signs, markings, flags, lights and flagmen as may be required by the engineer for the information and protection of traffic. All such measures should be as per requirement of MOST. | MOST: 112.4 MOST: 112.1 | During construction | Contractor/PMC | PIA |
| Occupational Health and Safety | The contractor is required to comply with all the precautions as required for the safety of workmen as per the International Labour Organisation (ILO) Convention No. 62, as far as those are applicable to the contract. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |

# TABLE 7.8 Cont..

# CONSTRUCTION STAGE EMP (CST PEDESTRIAN SUBWAY PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
|  | The contractor shall supply all necessary safety appliances such as safety goggles, helmets, safety belts, ear plugs, masks etc. to the worker and staff. All laws related to safe scaffolding, ladders, working platform, gangway, stairwells, excavations, safety entry and exit etc. shall be complied with. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |
|  | Adequate precautions shall be taken to prevent danger from electrical equipment. All machines/equipment used shall confirm to the relevant Indian Standards (IS) codes and shall be regularly inspected by the engineer. | MOST: 106 | During Construction | Contractor/PMC | PIA, MMRDA |
|  | A readily available first aid unit including an adequate supply of sterilized dressing material and appliances shall be provided as per the requirements under the Factory Act. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |

# TABLE 7.8 Cont..

# OPERATION STAGE EMP (CST PEDESTRIAN SUBWAY PROJECT)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Atmospheric pollution | Ambient air concentrations of various pollutants shall be monitored as per the pollution monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency - MCGB | MMRDA |
|  | Vehicle emission standards shall be enforced strictly. |  |  |  |  |
| Noise pollution | Monitoring of noise levels at various receptors as per monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency – MCGB RTO | MMRDA |
|  | Public awareness campaigns for noise reduction including placing adequate “No Honking” sign boards at sensitive locations. |  |  |  |  |
| Traffic and safety | Traffic management plans shall be prepared – Plans to restrict commercial vehicles in station area. | Project requirement | Through operation stage | Development Authorities, RTO | MMRDA |
|  | Traffic volume and speed to be monitored to record benefits achieved from the project. |  |  |  |  |

TABLE 7.9 Environmental Management Plan for Station Area Improvement Scheme (SATIS) at Thane West

# PRE-CONSTRUCTION / DESIGN STAGE

| Environmental Impact | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Traffic delays and congestion | Detailed traffic control plan shall be prepared prior to commencement of works on any section of the road. The control plans shall contain details of temporary diversions, details of arrangement of construction under traffic. | MOST: 112.1 | Before Construction | PIA | MMRDA |
|  | Special consideration shall be given in preparation of traffic control plan to the safety of pedestrians. | MOST: 112.2 |  | PIA | MMRDA |
| Relocation of Hawkers | Hawker relocation shall be carried out in accordance to the RAP and entitlement framework of the project. It shall be ensured that all R&R activities be reasonably completed as per RAP, before the construction activity starts | RAP Requirement | Before Start of construction | MMRDA | MMRDA |

# TABLE 7.9 Cont..

# CONSTRUCTION STAGE EMP (SATIS – THANE)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents |  | Time Frame |  | Responsibility |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Implementation |  |  | Supervision |
| Generation of Dust due to construction activity | All vehicles carrying construction material shall be covered to avoid material spillage. While unloading material, fall height shall be kept low to minimize fugitive dust generation. | MOST:111.9 |  | During construction |  | Contractor/PMC |  |  | PIA, MMRDA |
|  | Water sprinkling to minimize dust from excavated material. | Contract Documents |  |  |  |  |  |  |  |
|  | Construction site to be suitably enclosed to contain fugitive dust emissions. |  |  |  |  |  |  |  |  |
|  | All Earthwork and construction material should protected in such a manner so as to minimize generation of dust. |  |  |  |  |  |  |  |  |
| Construction Vehicle/equipm-ent exhaust | Exhaust emissions from all construction equipment (DG sets, compressors etc.) shall adhere to emission norms. | Contract requirement |  | During construction |  | Contractor/PMC |  |  | PIA, MMRDA |
| Noise from vehicles, plants and equipment Noise from vehicles, plants and equipment | All vehicle and construction equipment shall be fitted with exhaust silencers. Damaged silencers to be promptly replaced by contractor. | MOST 111.1 |  | During construction |  |  |  |  |  |
|  | All construction equipment (particularly DG sets, shall adhere to noise standards of Ministry of Environment and Forests. | MOST:111.13 |  |  |  |  |  |  |  |
|  | Operation of noise generating equipment such as pile driving, concrete and drilling etc. shall be done during night time as this area is predominantly pubic and commercial where day time activity is more than night time. | Contract requirement |  |  |  |  |  |  |  |
|  | Workers exposed to loud noise (As per Factory Act requirements) shall wear earplugs/earmuffs |  | MOST: 111.6 |  | During construction |  | Contractor/PMC | PIA, MMRDA |  |

# TABLE 7.9 Cont..

# CONSTRUCTION STAGE EMP (SATIS – THANE)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Debris disposal | Debris generated from due to any dismantling of existing pavement/structures shall be suitably reused in proposed construction. Unutilisable debris shall be suitably disposed as fill material at an approved location elsewhere. | MOST: 112.6 MOST :112.2 | During construction | Contractor/PMC | PIA, MMRDA |
| Traffic delays and congestion | Detailed traffic control plan shall be prepared prior to commencement of works on any section of the road. The control plans shall contain details of temporary diversions, details of arrangement of construction under traffic. | MOST: 112.1 | Before Construction | PIA | MMRDA |
|  | Special consideration shall be given in preparation of traffic control plan to the safety of pedestrians. | MOST: 112.2 |  | PIA | MMRDA |
| Traffic Control and Safety | The contractor shall take all necessary measures for the safety of traffic during construction and provide, erect and maintain such barricades, including signs, markings, flags, lights and flagmen as may be required by the engineer for the information and protection of traffic. All such measures should be as per requirement of MOST. | MOST: 112.4 MOST: 112.1 | During construction | Contractor/PMC | PIA |
| Occupational Health and Safety | The contractor is required to comply with all the precautions as required for the safety of workmen as per the International Labour Organisation (ILO) Convention No. 62, as far as those are applicable to the contract. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |

# TABLE 7.9 Cont..

# CONSTRUCTION STAGE EMP (SATIS – THANE)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
|  | The contractor shall supply all necessary safety appliances such as safety goggles, helmets, safety belts, ear plugs, masks etc. to the worker and staff. All laws related to safe scaffolding, ladders, working platform, gangway, stairwells, excavations, safety entry and exit etc. shall be complied with. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |
|  | Adequate precautions shall be taken to prevent danger from electrical equipment. All machines/equipment used shall confirm to the relevant Indian Standards (IS) codes and shall be regularly inspected by the engineer. | MOST: 106 | During Construction | Contractor/PMC | PIA, MMRDA |
|  | A readily available first aid unit including an adequate supply of sterilized dressing material and appliances shall be provided as per the requirements under the Factory Act. | Contract Requirement | During Construction | Contractor/PMC | PIA, MMRDA |

# TABLE 7.9 Cont..

# OPERATION STAGE EMP (SATIS – THANE)

| Environmental Impact/Issue | Mitigation Measures | Reference to Contract Documents | Time Frame | Responsibility |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Implementation | Supervision |
| Atmospheric pollution | Ambient air concentrations of various pollutants shall be monitored as per the pollution monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency - MCGB | MMRDA |
|  | Vehicle emission standards shall be enforced strictly. |  |  |  |  |
| Noise pollution | Monitoring of noise levels at various receptors as per monitoring plan. | Project requirement | Starting immediately after completion of construction | Pollution monitoring agency – Thane Municipal Corporation. RTO, PIA | MMRDA |
|  | Public awareness campaigns for noise reduction including placing adequate “No Honking” sign boards at sensitive locations. |  |  |  |  |
| Traffic and safety | Traffic management plans shall be prepared – Plans to restrict commercial vehicles in station area. | Project requirement | Through operation stage | Development Authorities, RTO | MMRDA |
|  | Traffic density to be monitored to record benefits achieved from the project. |  |  |  |  |
|  | Provide pathways and crossings for pedestrians. |  |  |  |  |

Table 7.10	Project Level Environmental Monitoring Plan – JVLR PROJECT

| Environmental Component | Project Stage | Parameters | Standard | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| Air Quality | Construction | SPM, PM10, SO2, N0x | NAAQS See Appendix 7.1 | -Fantasy Land -L&T -IIT Main gate -Gandhi Nagar Square | Once every season –Summer, Winter, post-monsoon | 24 hr/day for 2 consecutive working days per week for 2 weeks. | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  |  | CO, HC | NAAQS See Appendix 7.1 | -Fantasy Land -L&T -IIT Main gate -Gandhi Nagar Square | Once every season –Summer, Winter, post-monsoon | 8 hr/day for 2 consecutive working days per week for 2 weeks. | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | SPM, PM10, SO2, N0x | NAAQS See Appendix 7.1 | -Fantasy Land -L&T -IIT Main gate -Gandhi Nagar Square | Once every season -Summer, Winter, post-monsoon for 1 year after operation starts. | 24 hr/day for 2 consecutive working days per week for 2 weeks. | MCGM | MMRDA |
|  |  | CO, HC | NAAQS See Appendix 7.1 | -Fantasy Land -L&T -IIT Main gate -Gandhi Nagar Square | Once every season -Summer, Winter, post-monsoon for 1 year after operation starts. | 8 hr/day for 2 consecutive working days per week for 2 weeks. | MCGM | MMRDA |

Table 7.10 Cont.  Environmental Monitoring Plan – JVLR PROJECT

| Environmental Component | Project ### Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| ### Noise level | Construction | Leq, L10, L50, L90 dB(A) | CPCB noise standards | -Fantasy Land -L&T -IIT Main gate -Gandhi Nagar Square And at sensitive and residential locations located near construction equipment.. | At start of construction activity, followed every season during construction period | Continuous 24 hour reading with a frequency of 10 minutes | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | Leq, L10, L50, L90 dB(A) | CPCB noise standards | -Fantasy Land -L&T -IIT Main gate -Gandhi Nagar Square | Once every season (excluding monsoon) for 1 year after operation starts. | Continuous 24 hour reading with a frequency of 10 minutes | MCGM | MMRDA |
| Soil Quality | Construction | Heavy metals and Oil and grease | Contaminant threshold level given by USEPA | -Fantasy Land -L&T -Gandhi Nagar Square | At start of construction activity, followed every season during construction period | One time sample | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | Heavy metals and Oil and grease |  | Only at accidental spill sites | -- | -- | MCGM | MMRDA |

| Environmental Component | Project ### Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Plantation including endangered species | Pre-Construction | Monitoring of tree felling. | As laid out in project detail design. Trees to be adequately marked for felling. | At ecologically sensitive locations such as mangrove areas. | During tree felling | -- | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | Survival rate of road side plantation and other compensatory plantation. | Survival rate to be al-least 70%. Below which re-plantation should be done. | At locations of compensatory plantation | Annual | For 3 years after operation starts | PIA / Contractor | MMRDA |
| Traffic Volume | Operation | Traffic volume ,characteristic and speed. | -- | -L&T western section -Near IIT on middle section -Near Gandhi Nagar Square on EEH. | 3 day hourly counts | Annual | MMRDA | MMRDA |

Table 7.11	Project Level Environmental Monitoring Plan – BVQR PROJECT

| Environmental Component | Project ### Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| Water Quality | Construction | PH, BOD, COD, TDS, DO, Oil and grease. | CPCB water quality standards | Only for affected creek water locations near 15 bridges. Locations to be finalized in updated project EA. | Once every 6 months during construction period | One time sample | Contractor through pre-approved monitoring agency | PIA, MMRDA |
| ### Noise level | Construction | Leq, L10, L50, L90 dB(A). (Day and Night) | CPCB noise standards | Between Borivali and Dahisar station - Between Mira road and Bhayandar station. - At sensitive location – near Ayurvedic college near Nalla Sopara. | At start of construction activity, followed every season during construction period. | Continuous 24 hour reading (1-day only) with a frequency of 30 minutes | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | Leq, L10, L50, L90 dB(A) (Day and Night) | CPCB noise standards | Same as above | Once every year after operation starts. | Continuous 24 hour reading (1-day only) with a frequency of 30 minutes | PIA / Western railway | MMRDA |

Table 7.11 Cont.	Environmental Monitoring Plan – BVQR PROJECT

| Environmental Component | Project ### Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| Soil Quality | Construction | Heavy metals and Oil and grease | Contaminant threshold level given by USEPA | At equipment yards. | At start of construction activity, followed every season during construction period | One time sample | Contractor /PMC | MMRDA |
|  | Operation | Heavy metals and Oil and grease |  | Only at accidental spill sites | -- | -- | Western Railway | MMRDA |
| Plantation including endangered species | Pre-Construction | Monitoring of tree felling. | As laid out in project detail design. Trees to be adequately marked for felling. | At ecologically sensitive locations such as mangrove areas. | During tree felling | -- | Contractor /PMC | MMRDA |
|  | Operation | Survival rate of track side plantation and other compensatory plantation. | Survival rate to be al-least 70%. Below which re-plantation should be done. | At all track side and compensatory plantation locations. | Annual | For 3 years after operation starts | Western Railway | MMRDA |

Table 7.12	Project Level Environmental Monitoring Plan – VIKHROLI ROB PROJECT

| Environmental Component | Project Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| Air Quality | Construction | SPM, PM10, N0x | NAAQS See Appendix 7.1 | Along ROB on Godrej Road at 2 m height and 5 m from road edge. | Once every season –Summer, Winter, post-monsoon | 24 hr/day for 2 consecutive working days per week for 1 week. | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  |  | CO, HC | NAAQS See Appendix 7.1 | Along ROB on Godrej Road at 2 m height and 5 m from road edge. | As above | 8 hr/day for 2 consecutive working days per week for 1 week. | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | SPM, PM10, N0x | NAAQS See Appendix 7.1 | Along ROB on Godrej Road at 2 m height and 5 m from road edge. | Once every season –Summer, Winter, post-monsoon for 1 year after operation starts. | 24 hr/day for 2 consecutive working days per week for 2 weeks. | PIA | MMRDA |
|  |  | CO, HC | NAAQS See Appendix 7.1 | Along ROB on Godrej Road at 2 m height and within 10 m from road edge. | Once every season –Summer, Winter, post-monsoon for 1 year after operation starts. | 8 hr/day for 2 consecutive working days per week for 2 weeks. | PIA | MMRDA |

Table 7.12 Cont.  Environmental Monitoring Plan – VIKHROLI ROB PROJECT

| Environmental Component | Project ### Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| ### Noise level | Construction | Leq, L10, L50, L90 dB(A) | CPCB noise standards | At sensitive and residential and sensitive receptors (2 schools and a church), within 100 m from road edge. | At start of construction activity, followed every season during construction period | Continuous 24 hour reading with a frequency of 10 minutes | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | Leq, L10, L50, L90 dB(A) | CPCB noise standards | As above | Once every season for 1 year after operation starts. | As above | PIA | MMRDA |
| Soil Quality | Construction | Heavy metals and Oil and grease | Contaminant threshold level given by USEPA | At equipment yards. | At start and completion of construction activity. | One time sample | Contractor through pre-approved monitoring agency | PIA, MMRDA |
| Plantation | Pre-Construction | Monitoring of tree felling. | As laid out in project detail design. Trees to be adequately marked for felling. | At tree felling location. | -- | -- | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | Survival rate of road side plantation and other compensatory plantation. | Survival rate to be al-least 70%. Below which re-plantation should be done. | At locations of compensatory plantation | Annual | For 3 years after operation starts | MCGM | MMRDA |

# Table 7.13	Project Level Environmental Monitoring Plan – (CST PEDESTRIAN SUBWAY PROJECT)

| Environmental Component | Project Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| Air Quality | Construction | SPM, PM10, N0x | NAAQS See Appendix 7.1 | At MCGM office opposite CST. | Once every month of construction (excluding monsoon season) | 24 hr/day for 1 working day. | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | SPM, PM10, N0x | NAAQS See Appendix 7.1 | At MCGM office opposite CST. | Once after operation starts, to serve as comparative data | 24 hr/day for 1 working day. | MCGM | MMRDA |
|  |  | CO, HC | NAAQS See Appendix 7.1 | At MCGM office opposite CST. | Once after operation starts, to serve as comparative data | 8 hr/day for 1 working day. | MCGM | MMRDA |

Table 7.13 Cont.  Environmental Monitoring Plan – (CST PEDESTRIAN SUBWAY PROJECT)

| Environmental Component | Project ### Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| ### Noise level | Construction | Leq, L10, L50, L90 dB(A) | CPCB noise standards | At same site as AAQ monitoring station. | Once every month of construction | Continuous 24 hour reading with a frequency of 10 minutes | Contractor through pre-approved monitoring agency | PIA, MMRDA |
|  | Operation | Leq, L10, L50, L90 dB(A) | CPCB noise standards | As above | Once after operation starts, to serve as comparative data with baseline | As above | MCGM | MMRDA |
| Traffic Congestion | Operation | Traffic volume, composition and speed. | -- | Near subway | Once in six months for the 1st year after operation starts. | 12- hr (Day Time) | RTO | MMRDA |

# Table 7.14	Project Level Environmental Monitoring Plan – (SATIS)

| Environmental Component | Project Stage | Parameters | Standards | Location | Frequency | Duration | Institutional Responsibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Implementation | Supervision |
| Air Quality | Operation | SPM, PM10, N0x | NAAQS See Appendix 7.1 | Same as baseline monitoring location at station complex. | Every three months for 1 year after SATIS is complete. This will serve as comparative data to baseline values | 24 hr/day for 1 working day. | MCGM | MMRDA |
|  |  | CO, HC | NAAQS See Appendix 7.1 | Same as baseline monitoring location at station complex. | Every three months for 1 year after SATIS is complete. This will serve as comparative data to baseline values | 8 hr/day for 1 working day. | MCGM | MMRDA |
| ### Noise level | Operation | Leq, L10, L50, L90 dB(A) | CPCB noise standards | At air monitoring locations. | Every three months for 1 year after SATIS is complete. This will serve as comparative data to baseline values | As above | MCGM | MMRDA |
| Traffic Congestion | Operation | Traffic volume, composition and speed. | -- | At the 3 baseline monitoring locations | Once in six months for the 1st year after SATIS is complete. | 12- hr (Day Time) | RTO | MMRDA |

TABLE 7.15 	MONITORING AND REPORTING PROCESS

| Construction Stage | Reporting Parameter | Reporting Format Number | Contractor Reporting Frequency to PMC | PMC Reporting Frequency to PIA | PIA reporting Frequency to MMRDA | MMRDA reporting Frequency to World Bank | World Bank desired supervision |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-Construction | Trees to be removed |  | Monthly | Monthly | Quarterly | Quarterly | -- |
|  | PAH R&R |  | -- |  | Monthly | Monthly | Half-yearly |
|  | Relocation of utility and community resources |  | Monthly | Monthly | Quarterly | Quarterly |  |
| Construction | Fugitive dust mitigation |  | Daily | Monthly | Monthly | Quarterly | Half-yearly |
|  | Condition of construction equipment w.r.t noise and emissions |  | Daily | Monthly | Monthly | Quarterly | Half-yearly |
|  | Identification of soil erosion locations and measures taken |  | One time | One time | One time | -- | -- |
|  | Road –side and compensatory plantation schedule |  | Monthly | Monthly | Quarterly | Quarterly | Half-yearly |
|  | Plantation survival rate reporting |  | Monthly | Monthly | Quarterly | Quarterly | Half-yearly |
|  | Pollution monitoring |  | Monthly | Monthly | Monthly | Quarterly | Half-yearly |
|  | Debris disposal plan/locations. |  | Weekly | Monthly | Quarterly | Quarterly | Half-yearly |

TABLE 7.15 	Cont..

| Construction Stage | Reporting Parameter | Reporting Format Number | Contractor Reporting Frequency to PIA | PMC Reporting Frequency to PIA | PIA reporting Frequency to MMRDA | MMRDA reporting Frequency to World Bank | World Bank desired supervision |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Construction | Top soil Management |  | Monthly | Quarterly | Quarterly | Quarterly | -- |
|  | Quarry identification and management |  | Monthly | Monthly | Quarterly | Quarterly | Half yearly |
|  | Occupational safety and health |  | Weekly | Monthly | Monthly | Monthly | Half yearly |
|  | Labour camp set-up and hygiene. |  | Weekly | Monthly | Monthly | Monthly | Half yearly |
|  | Road safety measures |  | Weekly | Monthly | Monthly | Monthly | Half yearly |
|  | Site enhancement implementation plan at water bodies, road junctions etc. |  | Monthly | Quarterly | Quarterly | Quarterly | Half yearly |
| Operation | Pollution monitoring schedule and reports |  | -- | -- | Quarterly | Quarterly | Half-yearly |

Table 7.16	ROLES/RESPONSIBILITIES AND STRENGTHENING OF INSTITUTIONS

| # ORGANISATION | ROLES & RESPONSIBILITIES | STRENGTHING REQUIRED |
| --- | --- | --- |
| ## (Mumbai Metropolitan Region Development Authority (MMRDA) | Review of implementation of Environmental Mitigation Measures (EMPs) for MUTP projects. Ensure adequate Resettlement and Rehabilitation of PAH Facilitate implementation of policy directives/ emission laws etc. for pollution prevention/mitigation by interacting with various the government departments like Environment Department, Urban Development Department, RTO etc. Review the environmental management capabilities of implementing agencies, particularly municipal authorities and to assist them in developing their capabilities. Obtain and analyse environmental information generated by organizations like MCGM, MPCB, RTO etc. and factor them into short term and long term planning process for overall sustainable development of MMR. | Enhance the capabilities of the existing “Environmental Cell” of MMRDA by out sourcing whenever required. Training needed on: - Environmental assessment, social impacts. - Appreciation of Environmental impacts and EMPs identified for related MUTP projects. Procedure and responsibilities for EMP implementation, monitoring and reporting etc. |
| Municipal Corporation of Greater Mumbai (MCGB) | Monitoring of ambient air quality and noise at existing locations. Extending monitoring network to MUTP project locations for post-project monitoring. Regular report to MMRDA to enable environmental planning at a regional level. | MCGM is operating air quality monitoring network for last several years and already have trained personnel. Training in relation to Quality Assurance, data analysis and dissemination and other issues as suggested in the report ”Study for Strengthening Air Quality Monitoring network of MCGM, Nov. 2000) However the existing staff and monitoring equipment need to be upgraded for the additional monitoring load due to MUTP project. |

Table 7.16  Cont..

| # ORGANISATION | ROLES & RESPONSIBILITIES | STRENGTHING REQUIRED |
| --- | --- | --- |
| Public Works Department (PWD), Maharashtra State Road Development Corporation (MSRDC) | Ensuring implementation of EMPs for related MUTP projects through PMC/contractors Carrying out Environmental Assessments of all related on-going and future Transport Projects. | Environmental appraisal capabilities of existing staff to be enhanced through training programs. Short term module type training programs needed for: - Environmental assessment, social impacts. - Appreciation of Environmental impacts and EMPs identified for related MUTP projects. Procedure and responsibilities for EMP implementation, monitoring and reporting etc. |
| # Railways/Mumbai Rail Vikas Corporation (MRVC) | Ensuring implementation of EMPs for related MUTP projects through PMC/contractors Carrying out Environmental Assessments of all related on-going and future Transport Projects. | Environmental appraisal capabilities of existing staff to be enhanced through training programs. Short term module type training programs needed for: - Environmental assessment, social impacts. - Appreciation of Environmental impacts and EMPs identified for related MUTP projects. Procedure and responsibilities for EMP implementation, monitoring and reporting etc. |
| ### Transportation Service Organisations (BEST) | Ensuring regular emission and maintenance checks on the bus fleet to ensure compliance with regulations. Good housekeeping in existing Bus-Depots to minimize water and land pollution from oil spills and waste oil disposal. | Emission monitoring capabilities of the bus fleet to be strengthened. Environmental awareness training of key personnel |
| Enforcement/ Regulatory Organizations: Regional Transport Office (RTO) | Enforcement of vehicular emission standards, with more emphasis on heavy vehicles, taxies and 3 wheelers | Environmental awareness training for its vigilance staff so that they can appreciate the importance of their role and carry out the same diligently. Training also needs to include criteria and techniques for vehicle inspection and certification. |